Design, Operation and Optimization of Constructed Wetland for Removal of Pollutant

Manuscript ID: ijerph-958693

Thank you very much for the suggestions and comments made by individual reviewers. Following are the line by line responses to the reviewer 3. Here is noted that corrections are highlighted in main text in Green colour.

Response to the Comments of Reviewer 3

1) Original line no.: 19-32

Line no., after correction: 19-36

Reviewer comments: This article shows a review provides a detailed review of the implementation of constructed wetlands, designs, operation, optimization, and yields and their relationship to plant varieties, substrates, bed depth, hydraulic head, to give a clearer picture of the operation and challenges faced by technology in wastewater management. Although it is an important documentary study, I consider it necessary to make some considerable changes before continuing with its processing and future publication, as described below, I would like to be able to see the document once I have the observations taken care of since I consider it important if you note to give a different perspective and punctual to the future scholar.

The abstract should integrate important findings from the desk research.

Corrections are made by Authors: We have already integrated the abstract with important findings from the desk research.

2) Original line no.: 35-83 and 50-57

Line no., after correction: 39-118 and 77-93

Reviewer comments: From page 35 to 83, although I consider the introduction good, it lacks new references, there are no references, new not even from 2018, 2019, and 2020, where technology has evolved, on the other hand, lines 50 to the 57, mentions that, in the last decade, and references dates much more distant from these than a decade, I consider that the document should review more recent studies, in general, strengthen this part with new references of key studies.

Corrections are made by Authors: We have strengthened the Introduction Part with recent references, which is marked below in green color for your information.

Fitton, N.; Alexander, P.; Arnell, N.; Bajzelj, B.; Calvin, K.; Doelman, J.; Gerber, J.S.; Havlik, P.; Hasegawa, T.;
Herrero, M.; et al. The vulnerabilities of agricultural land and food production to future water scarcity. Glob.
Environ. Chang. 2019, 58, 101944. [CrossRef]

Jiang, L.; Guli·Jiapaer; Bao, A.; Yuan, Y.; Zheng, G.; Guo, H.; Yu, T.; De Maeyer, P. The effects of water stress
on croplands in the Aral Sea basin. J. Clean. Prod. 2020, 254, 120114. [CrossRef]

UNESCO. The United Nations World Water Development Report—Leaving no One Behind; UN iLibrary: Paris,
France, 2019.

EC. Water is Too Precious to Waste. Available online: https://ec.europa.eu/environment/water/pdf/water_
reuse_factsheet_en.pdf (accessed on 1 February 2020)

Cui, R.Y.; Calvin, K.; Clarke, L.; Hejazi, M.; Kim, S.; Kyle, P.; Patel, P.; Turner, S.; Wise, M. Regional responses
to future, demand-driven water scarcity. Environ. Res. Lett. 2018, 13, 094006. [CrossRef]

Garrote, L.; Iglesias, A.; Granados, A. Country-level assessment of future risk of water scarcity in Europe.
In Proceedings of the International Association of Hydrological Sciences, Beijing, China, 13–15 June 2018;
Volume 379, pp. 455–462.

Suárez-Almiñana, S.; Pedro-Monzonís, M.; Paredes-Arquiola, J.; Andreu, J.; Solera, A. Linking Pan-European
data to the local scale for decision making for global change and water scarcity within water resources
planning and management. Sci. Total Environ. 2017, 603–604, 126–139.

Gampe, D.; Nikulin, G.; Ludwig, R. Using an ensemble of regional climate models to assess climate change
impacts on water scarcity in European river basins. Sci. Total Environ. 2016, 573, 1503–1518. [CrossRef]

EEA. Is Europe’s Freshwater Use Sustainable? Available online: https://www.eea.europa.eu/highlights/
world-water-day-is-europe (accessed on 1 February 2020a).

EEA. Water Scarcity. Available online: https://www.eea.europa.eu/archived/archived-content-water-topic/
featured-articles/water-scarcity (accessed on 1 February 2020b).

Elbana, T.A.; Bakr, N.; Elbana, M. Reuse of treated wastewater in Egypt: Challenges and opportunities.
Handb. Environ. Chem. 2019, 75, 429–453.

Baawain, M.; Almamun, M.; Omidvarborna, H.; Al-Sabti, A.; Choudri, B.S. Public perceptions of reusing
treated wastewater for urban and industrial applications: Challenges and opportunities. Environ. Dev.
Sustain. 2018, 22, 1–13. [CrossRef]

Collivignarelli, M.C.; Carnevale Miino, M.; Baldi, M.; Manzi, S.; Abbà, A.; Bertanza, G. Removal of non-ionic
and anionic surfactants from real laundry wastewater by means of a full-scale treatment system. Process Saf.
Environ. Prot. 2019a, 132, 105–115. [CrossRef]

Collivignarelli, M.C.; Abbà, A.; Carnevale Miino, M.; Arab, H.; Bestetti, M.; Franz, S. Decolorization and
biodegradability of a real pharmaceutical wastewater treated by H2O2-assisted photoelectrocatalysis on
TiO2 meshes. J. Hazard. Mater. 2019b, 387, 121668. [CrossRef] [PubMed]

Collivignarelli, M.C.; Abbà, A.; Benigna, I. The reuse of biosolids on agricultural land: Critical issues and
perspective. Water Environ. Res. 2019c, 1196. [CrossRef] [PubMed]

Fu, Q.; Malchi, T.; Carter, L.J.; Li, H.; Gan, J.; Chefetz, B. Pharmaceutical and personal care products: From
wastewater treatment into agro-food systems. Environ. Sci. Technol. 2019, 53, 14083–14090. [CrossRef]
[PubMed]

Egea-Corbacho Lopera, A.; Gutiérrez Ruiz, S.; Quiroga Alonso, J.M. Removal of emerging contaminants
from wastewater using reverse osmosis for its subsequent reuse: Pilot plant. J. Water Process Eng. 2019, 29,
100800. [CrossRef]

Macías-García, A.; García-Sanz-Calcedo, J.; Carrasco-Amador, J.P.; Segura-Cruz, R. Adsorption of paracetamol
in hospital wastewater through activated carbon filters. Sustainability 2019, 11, 2672. [CrossRef]

Cheng, G., Li, Q., Su, Z., Sheng, S., Fu, J., 2018. Preparation, optimization, and
application of sustainable ceramsite substrate from coal fly ash/waterworks
sludge/oyster shell for phosphorus immobilization in constructed wetlands. J. Clean.
Prod. 175, 572-581.

Lutterbeck, C.A., Machado, E.L., Sanchez-Barrios, A., Silveira, E.O., Layton, D., Rieger, A., Lobo, E.A., 2020. Toxicity evaluation of hospital laundry wastewaters treated by microbial fuel cells and constructed wetlands. Sci. Total Environ. 729, 138816.

Fu, G., Wu, J., Han, J., Zhao, L., Chan, G., Leong, K.J., 2020. Effects of substrate type on denitrification efficiency and microbial community structure in constructed wetlands. Bioresour. Technol. 307, 123222.

Tang, C., Zhao, Y., Kang, C., Yang, Y., Morgan, D., Xu, L., 2019. Towards concurrent pollutants removal and high energy harvesting in a pilot-scale CW-MFC: Insight into the cathode conditions and electrodes connection. Chem. Eng. J. 373, 150–160.

3) Original line no.: 116-117

Line no., after correction: 154-171

Reviewer comments: In section 4, lines 116 to 117, in the specific case of Mexico, some studies have used PET, porous stone from rivers, and mixtures of substrates in constructed wetlands that I consider should also be part of these new materials to be unblocked because they are recycled and have a different view of the studies and will give an overview of the use of these substrates in new designs.

Corrections are made by Authors: An overview of the use of those substrates in new designs are given as follows.

According to CONAGUA (CONAGUA, 2019), of municipal collected wastewater in Mexico, only the 63% of this is treated, which shows the clear need for wastewater treatment plants; however, the implementation of this is not a common case due the high costs required for the construction and operation thereof. Therefore, the use of ecologically viable or sustainable alternatives to solve these problems is needed; for this purpose, constructed wetlands (CWs), or so-called artificial wetlands are examples of such an alternative (Mitsch and Gosselink, 2015; Wu et al., 2015; García-García, Ruelas-Monjardín and Marín-Muñíz 2016). Improving water quality is a relevant environmental aspect, and using constructed wetlands (CWs) is a sustainable option for this; both porous materials filled cells and plants that collectively remove contaminants must be readily available and inexpensive. This study evaluated CWs and their functionality by comparing two ornamental plants (Spathiphyllum wallisii and Hedychium coronarium) planted in experimental mesocosm units filled with layers of porous river rock, tepezil, and soil, or in mesocosms with layers of porous river rock, and tepezil, without the presence of soil. The findings during the experiments (180 days), showed that the removal of pollutants (chemical oxygen demand (COD), total solids suspended (TSS), nitrogen as ammonium (N-NH4), as nitrate (N-NO3), and phosphate (P-PO4) was 20–50% higher in mesocosms with vegetation that in the absence of this, and those mesocosms with the soil layer between 33–45% favored removal of P-PO4. Differences regarding of vegetation removal were only observed for N-NH4, being 25–45% higher in CWs with H. coronarium, compared with S. wallisii (Zamora et al., 2019).

4) Original line no.: 122-137

Line no., after correction: 191-222

Reviewer comments: From pages 122 to 137, they talk about plant substrate interaction, however, there is no new survey of ornamental plants in wetlands, which are more recent studies, consult these documents that have a description of plants in wetlands and their plant substrate interaction (Role of wetland plants and use of ornamental flowering plants in constructed wetlands for wastewater treatment: a review.)

Corrections are made by Authors: We have enriched with the description of ornamental plants in wetlands and their plant substrate interaction which are as follows.

The current knowledge about the role terrestrial ornamental plants play in constructed wetlands (CWs) has scarcely been evaluated. Likewise, little attention has been given towards the use of new support or fill media for subsurface flow CWs, which may result in the reduction of costs when implemented on a large scale. This study evaluated, during nine months, the effect of three terrestrial ornamental plants and two substrates on the elimination of pollutants in wastewaters by using fill-and-drain vertical subsurface flow CWs (FD-CWs). Sixteen microcosms were used, nine filled with polyethylene terephthalate (PET) and nine with porous river stone (PRS). For each type of substrate, duplicates of microcosms were used, utilizing Anthurium sp., Zantedeschia aethiopica, and Spathiphyllum wallisii as vegetation and two other CWs without vegetation as controls. The environmental conditions, number of flowers, and height of the plants were registered. The results revealed that both substrates in the FD-CWs were efficient in removing pollutants. The average removal of pollutants in systems with vegetation revealed a positive effect on the reduction of the biochemical oxygen demand (55–70%), nitrates (28–44%), phosphates (25–45%), and fecal coliforms (52–65%). Meanwhile, in units without vegetation, the reduction of pollutants was nearly 40–50% less than in those with vegetation. The use of PET as a filling substrate in CWs did not affect the growth and/or the flowering of the species (Sandoval et al., 2019a).

The vegetation in constructed wetlands (CWs) plays an important role in wastewater treatment. Popularly, the common emergent plants in CWs have been vegetation of natural wetlands. However, there are ornamental flowering plants that have some physiological characteristics similar to the plants of natural wetlands that can stimulate the removal of pollutants in wastewater treatments; such importance in CWs is described here. A literature survey of 87 CWs from 21 countries showed that the four most commonly used flowering ornamental vegetation genera were Canna, Iris, Heliconia and Zantedeschia. In terms of geographical location, Canna spp. is commonly found in Asia, Zantedeschia spp. is frequent in Mexico (a country in North America), Iris is most commonly used in Asia, Europe and North America, and species of the Heliconia genus are commonly used in Asia and parts of the Americas (Mexico, Central and South America). This review also compares the use of ornamental plants versus natural wetland plants and systems without plants for removing pollutants (organic matter, nitrogen, nitrogen and phosphorous compounds). The removal efficiency was similar between flowering ornamental and natural wetland plants. However, pollutant removal was better when using ornamental plants than in unplanted CWs. The use of ornamental flowering plants in CWs is an excellent option, and efforts should be made to increase the adoption of these system types and use them in domiciliary, rural and urban areas (Sandoval et al., 2019b).

5) Original line no.: 145-157

Line no., after correction: 284-313

Reviewer comments: Section 6. It requires a definition of emerging countries according to who, and if it is possible to mention them, would be friendlier to the reader. On the other hand, stop when talking about these countries, it would be interesting to see some studies developed in them and analyze the latent need for water treatment in them where the constructed wetlands represent a high alternative for solving wastewater treatment problems.

Corrections are made by Authors: We have included the below text for correction.

Inadequate access to clean water and sanitation has become one of the most pervasive problems affecting human health in developing countries, and problems with water are expected to worsen in coming decades (Hutton et al., 2007; Shannon et al., 2008). Developing countries are defined according to their Gross National Income (GNI) per capita per year. Countries with a GNI of US$ 12,615 and less are defined as developing (World Bank, 2012). List of developing countries gave in the Table-2 (According to World Bank, 2012).

Table-2: List of developing countries (According to World Bank, 2012).

| Upper middle income (between $4,086 and $12,615 GNI per capita) | Lower middle income (between $1,036 and $4,085 GNI per capita) | Low-income (less than $1,035 GNI per capita) |
| --- | --- | --- |
| Albaniab, Algeria, Angola, Argentina, Azerbaijan, Belarus, Bosnia and Herzegovina, Botswana, Brazil, Bulgaria, China, Colombia, Costa Rica, Cuba, Dominican Republic, Ecuador, Gabon, Hungary, Iran Islamic Republic, Iraq, Jamaica, Jordan, Kazakhstan, Lebanon, Libya, Malaysia, Mauritius, Mexico, Montenegro, Namibia, Panama, Peru, Romania, Serbia, South Africa, Thailand, The former Yugoslav Republc of Macedonia, Tunisia, Turkey, Turkmenistan, Venezuela, RB. | Armenia, Bolivia, Cameroon, Cape Verde, Congo, Côte d’Ivoire, Djibouti, Egypt, El Salvador, Georgia, Ghana, Guatemala, Guyana, Honduras, India, Indonesia, Lesotho, Mauritania, Moldova, Morocco, Nicaragua, Nigeria, Pakistan, Papua New Guinea, Paraguay, Philippines, São Tomé and Principe Senegal, Sri Lanka, Sudan, Syrian Arab Republic, Ukraine, Uzbekistan, Vietnam, Yemen Rep., Zambia. | Bangladesh, Benin, Burkina Faso, Burundi, Central African Republic, Chad, Comoros, Democratic Republic of the Congo, Eritrea, Ethiopia, Gambia, The Guinea Guinea-Bissau, Haiti, Kenya, Kyrgyz Republic, Liberia, Madagascar, Malawi, Mali, Mozambique, Myanmar, Nepal, Niger, Rwanda, Sierra Leone, Somalia, Tajikistan, Tanzania, Togo, Uganda, Zimbabwe. |

According to a recent report by World Health Organization (2012), more than one-tenth of the global population (780 million) still relied on sub-standard drinking water sources in 2010. Lack of sanitation is an even larger concern. An estimated 2.5 billion people are still without improved sanitation, and sanitation coverage is below 50% in many countries of Sub-Saharan Africa and Southern Asia. Consequently, millions of people die annually (3900 children per day) from diseases transmitted through unsafe water or human excreta (Shannon et al., 2008). With insufficient water resources to meet rising water demand, many sources of water (e.g., groundwater) that are considered easy to be developed geographically and technologically have been overexploited in many developing countries (Llamas and Martínez- Santos, 2005; Jiang, 2009). This short-term strategy is likely to have detrimental effects on the environment, such as ground subsidence, salinity intrusion, and ecosystem deterioration (Kivaisi, 2001). In addition, many cities in developing countries have also generally fallen behind in constructing and managing sewage treatment facilities. Among the various developments, treatment of wastewater is always considered one of the lowest priorities (Konnerup et al., 2011). The consequence of this is the common practice of discharging large amounts of untreated wastewater directly into streams and lakes in many developing countries (Senzia et al., 2003; Shrestha et al., 2001). Ecological technologies such as constructed wetlands for wastewater treatment represent innovative and emerging solutions for environmental protection and restoration, placing them in the overall context of the need for low-cost and sustainable wastewater treatment systems in developing countries (Babatunde et al., 2008; Vymazal, 2011).

6) Original line no.: 203-222

Line no., after correction: 393-435

Reviewer comments: Jan section 7.1.2. I do not see information on ornamental plants that resist pollutant conditions, as well as mention the future direction of studies in this line of action. Reference must be made in the evolution of larger standing plants that can contribute more notably to the elimination of pollutants. In 266 to 269, review the document of ornamental plants referred to above in the recommendations to give a broader criterion of their use and potential in constructed wetlands (Role of wetland plants and use of ornamental flowering plants in constructed wetlands for wastewater treatment: a review)

Corrections are made by Authors: For improvement this jan section, We have included the following text.

7.1.2.1 Survey Results of the Use of Ornamental Flowering Plants in CWs

Many CWs around the world used OFP for the removal of various types of wastewater (Table 3). For example, in China, the most popular plants used is Canna sp., while in Mexico the ornamental plant used is more diverse, including plants with flowers of different colors, shapes and aromatic characteristics (Canna, Heliconia, Zantedeschia, Strelitzia spp).

Table 3. Ornamental flowering plants and removal of wastewater pollutants in CWs (constructed wetlands) around the globe.

| Country | Type of Wastewater | Vegetation | Removal Efficiency of Pollutants (%) | Reference |
| --- | --- | --- | --- | --- |
| Brazil | Domestic | Heliconia psittacorum | TSS: 88, COD: 95, BOD: 95 | Paulo et al., 2009 |
|  | Domestic | Alpinia purpurata, Arundina bambusifolia, Canna spp., Heliconia psittacorum L.F. | COD: 48-90, PO4 -P: 20, TKN: 31 and TSS: 34. | Paulo et al., 2013 |
|  | Swine | Hedychium coronarium, Heliconia rostrata | COD: 59, TP: 44, TKN: 34 and NHx 35 COD: 57, TP: 38, TKN: 34 and NHx: 37 | Sarmento et al., 2013 |
| Country | Type of Wastewater | Vegetation | Removal Efficiency of Pollutants (%) | Reference |
|  |  | Hemerocallis flava | COD: 72, BOD: 90, TN: 52, TP: 41 and SST: 72. | Prata et al., 2013 |
|  |  | Heliconia psittacorum L.F |  | Teodoro et al., 2014 |
| China | Municipal | Canna indica | COD: 77, BOD: 86, TP: >82, TN: >45 | Shi et al., 2004 |
|  | Aquaculture ponds | Canna indica mixed with other species | BOD: 71, TSS: 82, chlorophyll-a: 91.9, NH4 -N: 62, NO3 -N: 68 and TP: 20. | Li et al., 2007 |
|  | Domestic | Canna indica Linn | COD: 82.31, BOD: 88.6, TP: >80, TN: >85 | Yang et al., 2007 |
|  | Municipal | Canna indica | NH4 -N: 99, PO4 -P: 87 | Zhang et al., 2007 |
|  | Drain of some factories | R. carnea, I. pseudacorus, L. salicaria | COD: 58-92, BOD: 60-90 TN: 60-92, TP: 50-97, | Zhang et al., 2007 |
|  | River | Canna sp | COD: 95, N-NH4 : 100, N-NO3 : 76, TN: 72 | Sun et al., 2009 |
|  | Domestic | Canna indica | TP: 60, NH4 -N: 30-70, TN: ~25 | Cui et al., 2010 |
|  | Aquaculture ponds | Canna indica mixed with other natural wetland plants | BOD: 56, COD: 26, TSS: 58, TP: 17, TN: 48 and NH4 -N: 34. | Zhang et al., 2010 |
|  | Wastewater from a student dormitory (University) | Canna indica mixed with other natural wetland plants | COD: 50–70, BOD: 60–80, N-NO3 : 65–75, TP: 50–80 | Qiu et al., 2011 |
| China | Domestic | Canna indica and Hedychium coronarium | TP: 40–70 | Wen et al., 2011 |
|  | Polluted river | Iris pseudacorus mixed with other natural wetland plants | TN: 68, NH4 -N: 93, TP: 67 | Wu et al., 2011 |
|  | Sewage | Iris pseudacorus, mixed with other plants of natural wetlands | TN: 20 and TP: 44 | Xie et al., 2012 |
|  | Municipal | Canna indica | COD: 60, NO3 -N: 80, TN: 15, TP: 52 | Chang et al., 2012 |
|  | Simulated polluted river water | Iris sibirica | COD: 22, TN: 46, NH4 -N: 62, TP: 58 | Gao et al., 2014 |
|  | Synthetic | Canna sp | Fluoride: 51, Arsenic: 95 | Li et al., 2014 |
|  | Simulated polluted river water | Iris sibirica | Cd: 92 | Gao et al., 2015 |
| Country | Type of Wastewater | Vegetation | Removal Efficiency of Pollutants (%) | Reference |
|  | Synthetic | Canna indica L. | N: 56–60 | Hu et al. 2016 |
|  | Synthetic (hydrophonic sol.) | Canna indica L. | TN: 40–60, N-NO3: 20–95, NH4 -N: 20–55 | Wang et al., 2016 |
| Chile | Sewage | Zantedeschia aethiopica, Canna spp. and Iris spp | BOD: 82, TN: 53, TP: 60. | Morales et al., 2013 |
|  | Sewage | Tulbaghia violácea, and Iris pseudacorus. | BOD: 57–88, COD: 45–72, TSS: 70–93, PO4 -P: 6–20. | Burgos et al., 2017 |
|  | Ww rural community | Zantedeschia aethiopica | Organic matter: 60%, TSS: 90% | Leyva et al., 2018 |
| Colombia | Domestic | Heliconia psíttacorum | NH3 : 57 COD: 70 | Gutiérrez-Mosquera and Peña-Varón , 2011 |
|  | Synthetic landfill leachate | Heliconia psittacorum | COD, TKN and NH4 (all: 65–75) | Madera-Parra et al., 2015 |
|  | Cattle bath | Alpinia purpurata | SST: 58, TP: 85, COD: 63 | Marrugo-Negrete et al., 2016 |
|  | Municipal | Heliconia psitacorum | Bisphenol A: 73, Nonylphenols: 63 | Toro-Vélez et al., 2016 |
| Costa Rica | Dairy raw manure | Ludwigia inucta, Zantedechia aetiopica, Hedychium coronarium and Canna generalis | BOD: 62, NO3 -N: 93, PO4 -P: 91, TSS: 84 | León and Cháves, 2010 |
| Egypt | Municipal | Canna sp | TSS: 92, COD: 88, BOD: 90 | Abou-Elela and Hellal , 2012 |
|  | Municipal | Canna sp | TSS: 92, COD: 92, BOD: 92 | Abou-Elela et al., 2013 |
| India | Paper mill effluent | Canna indica | 9,10,12,13-tetrachlor- ostearic acid: 92 and 9,10-dichlorostearic acid: 96 | Choudhary et al., 2010 |
|  | Synthetic | Canna indica | Dye: 70-90 COD: 75 | Yadav et al., 2012 |
|  | Synthetic greywater | Heliconia angusta | COD:40, BOD: 70, TSS: 62, TDS: 19 | Saumya et al., 2015 |
|  | Domestic | Canna generalis | TN: 52, T-PO3: 9 | Ojoawo et al., 2015 |
|  | Collection pond | Canna Lily | BOD: 70-96, COD: 64–99 | Haritash et al., 2015 |
|  | Hostel greywater | Canna indica | COD, TKN and Pathogen all up 70 | Patil and Munavalli, 2016 |
|  | Domestic | Polianthus tuberosa L. | Heavy metals (Pb and Fe: 73–87), (Cu and Zn: 31–34) and Ni and Al: 20–26 | Singh and Srivastava, 2016 |
| Ireland | Domestic | Iris pseudacorus | TN: 30, TP:28 | Gill and O’Luanaigh, 2011 |
| Country | Type of Wastewater | Vegetation | Removal Efficiency of Pollutants (%) | Reference |
| Italy | Synthetic | Zantedeschia aethiopica, Canna indica | N: 65–67, P: 63–74, Zn and Cu: 98–99, Carbamazepine: 25–51, LAS: 60–72 | Macci et al., 2015 |
| Kenya | Flower farm | Canna spp. | BOD: 87, COD: 67, TSS: 90, TN: 61 | Kimani et al., 2012 |
| Mexico | Municipal | Zantedeschia aethiopoca | COD: 35, TN: 45.6 | Belmont and Metcalfe, 2003 |
|  | Domestic | Zantedeschia Aethiopica and Canna flaccid | SST: 85.9, COD: 85.8, NO3 -N: 81.7, NH4 -N: 65.5, NT: 72.6 | Belmont et al., 2004 |
|  | Coffee processing | Heliconia psittacorum | COD: 91, Coliformes: 93 | Orozco et al., 2006 |
|  | Domestic | Strelitzia reginae, Zantedeschia esthiopica, Canna hybrids, Anthurium andreanum, Hemerocallis Dumortieri | COD: >75, P: >66, Coliforms: 99 | Zurita et al., 2006 |
|  | Domestic | Zantedeschia aethiopica | BOD: 79, TN: 55, PT: 50 | Zurita et al., 2008 |
|  | Wastewater form canals | Zantedeschia aethiopica | COD: 92, N-NH4: 85, P-PO4 : 80 | Ramírez-Carrillo et al., 2009 |
|  | Municipal | Strelitzia reginae, Anthurium, andreanum. | TSS: 62, COD: 80, BOD: 82, TP: >50, TN: >49 | Zurita et al., 2009 |
|  | Groundwater | Zantedeschia aethiopica and Anemopsis californica | As: 75-78 | Zurita et al., 2009 |
|  | Domestic | Gladiolus spp | BOD: 33, TN: 53, TP: 75 | Castañeda and Flores, 2013 |
| Mexico | Mixture of greywater (from a cafeteria and research laboratories) | Zantedeschia aethiopica and Canna indica | COD: 65, NT: 22.4, PT: 5. | Zurita and White, 2014 |
|  | Domestic | Zantedeschia aethiopica | BOD: 70 | Hallack et al., 2018 |
|  | Domestic | Heliconia stricta, Heliconia psittacorum and Alpinia purpurata | BOD: 48, COD: 64, TP: 39, TN: 39 | Méndez-Mendoza et al., 2015 |
|  | Municipal | Canna hybrids and Strelitzia reginae | DQO: 86, NT: 30-33, PT: 24–44 | Merino-Solís et al., 2015 |
| Country | Type of Wastewater | Vegetation | Removal Efficiency of Pollutants (%) | Reference |
|  | Municipal | Zantedeschia aethiopica and Strelitzia reginae | COD: 75, TN: 18, TP: 2, TSS: 88. | Zurita and Carreón-Álvarez, 2015 |
|  | Domiciliar | Spathiphyllum wallisii, Zantedechia aethiopica, Iris japonica, Hedychium coronarium, Alocasia sp, Heliconia sp. and Strelitzia reginae | N-NH4 : 64–93 BOD: 22–96 COD: 25–64 | Garzón et al., 2016 |
|  | Community | Zantedeschia aethiopica, Lilium sp, Anturium spp and Hedychium coronarium | NT: 47, PT: 33, COD: 67 | Hernández, 2016 |
|  | Stillage Treatment | Canna indica | BOD: 87, COD: 70 | López-Rivera et al., 2016 |
|  | Artificial | Iris sibirica and Zantedeschia aethiopica | Carbamazepine: 50–65 | Tejeda et al., 2017 |
|  | Community | Alpinia purpurata and Zantedeschia aethiopica | --- | Marín-Muñiz et al., 2018 |
|  | Polluted river | Zantedeschia aethiopica | NO3 -N: 45, NH4 -N: 70, PO4 -P: 30 | Hernández et al., 2018 |
|  | Municipal | Spathiphyllum wallisii, and Zantedeschia aethiopica | --- | Sandoval-Herazo et al., 2018 |
|  | University | Strelitzia reginae | --- | Martínez et al., 2018 |
| Nepal | Municipal | Canna latifolia | TSS: 97, COD: 97, BOD: 89, TP: >30 | Sigh et al., 2009 |
| Portugal | Tannery | Canna indica mixed with other plants | COD: 41–73, BOD: 41–58 | Calheiros et al., 2007 |
|  | Community | Canna flaccida, Zantedeschia aethiopica, Canna indica, Agapanthus africanus and Watsonia borbonica | BOD, COD, P-PO4 , NH4 and total coliform bacteria (all up to 84) | Calheiros et al., 2015 |
| Spain | Domestic | Iris spp | Bacteria: 37 | García et al., 2008 |
|  | Municipal | Iris pseudacorus | Bacteria: 43 | Ansola et al., 2003 |
| Sri Lanka | Municipal | Canna iridiflora | BOD: 66, TP: 89, NH4 -N: 82, N-NO3 : 50 | Weragoda et al., 2012 |
| Taiwan | Domestic | Canna indica | N-NH4 : 73, BOD: 11 | Chyan et al., 2016 |
|  |  | Canna indica | N-NH4 : 57, N-NO3 : 57 | Chyan et al., 2016 |
| Thailand | Domestic | Canna spp | COD: 92, BOD: 93, TSS: 84, NH4 -N: 88, TP: 90 | Sirianuntapiboon and Jitvimolnimit, 2007 |
| Country | Type of Wastewater | Vegetation | Removal Efficiency of Pollutants (%) | Reference |
|  | Seafood | Canna siamensis, Heliconia spp and Hymenocallis littoralis | BOD: 91–99, SS: 52–90, TN: 72–92 and TP: 72–77 | Sohsalam et al., 2008 |
|  | Domestic | Heliconia psittacorum L.f. and Canna generalis L. Bailey | TSS: Both > 88, COD: 42–83 | Konnerup et al., 2009 |
|  | Fermented fish production | Canna hybrid | BOD, COD, TKN: ~ 97 | Kantawanichkul et al., 2009 |
|  | Collection system for business and hotel | Cannae lilies, Heliconia | BOD: 92, TSS: 90, NO3 -N: 50, TP: 46 | Brix et al., 2011 |
|  | Domestic | Crinum asiaticum, Spathiphyllum clevelandii Schott | PO4 -P: ~20 | Torit et al., 2012 |
| Turkey | Municipal | Iris australis | NH4 -N: 91, NO3 -N: 89, TN: 91 | Tunçsiper, 2009 |
| USA | Domestic | Canna flaccida, Gladiolus sp., Iris sp. | Baceria: ~50 | Neralla et al., 2000 |
|  | Nursery | Canna· generalis, Eleocharis dulcis, Iris Peltandravirginica. | N: ~50, P: ~60 | Palomsky et al., 2007 |
|  | Domestic | Iris pseudacorus L., Canna x. generalis L.H. Bail., Hemerocallis fulva L. and Hibiscus moscheutos L | BOD > 75, TSS > 88, Fecal baceteria > 93 | Karathanasis et al., 2003 |
|  | Tilapia production | Canna sp. | TSS: 90, NO2 -N: 91, NO3 -N: 76, COD: 12.5 and NH3 -N: 7.5 | Zachritz et al., 2008 |
|  | Stormwater runoff | Canna x generalis Bailey, Iris pseudacorus L., Zantedeschia aethiopica (L.) | N and P Canna (>90), Iris (>30) Zantedeschia (>90) | Chen et al., 2009 |
|  | Residential | Aeonium purpureum and Crassula ovate, Equisetum hyemale, Nasturtium, Narcissus impatiens, and Anigozanthos | TSS: 95 BOD: 97 | Yu et al., 2015 |
| Vietnam | Fishpond | Canna generalis | BOD: 50, COD: 25–55 | Konnerup et al., 2011 |
| United Kingdom | Herbicide polluted water | Iris pseudacorus | Atrazine: 90–100 | McKinlay and Kasperek, 1999 |

A review of the available literature showed that ornamental plants are used to remove pollutants from domestic, municipal, aquaculture ponds, industrial or farm wastewater. The removal efficiency of ornamental plants was also evaluated for the following parameters: biochemical oxygen demand (BOD), chemical oxygen demand (COD), total suspended solids (TSS), total nitrogen (TN), total phosphorous (TP), ammonium (NH4-N), nitrates (NO3-N), coliforms and some metals (Cu, Zn, Ni and Al). There is no clear pattern in the use of certain species of ornamental plants for certain types of wastewater. However, it is important to keep in mind that CWs using ornamental plants are usually utilized as secondary or tertiary treatments, due to the reported toxic effects that high organic/inorganic loading has on plants in systems that use them for primary treatment (in the absence of other complementary treatment options) (Gersberg et al., 1986; Duarte et al., 2010). The use of OFP in CWs generates an esthetic appearance in the systems. In CWs with high plant production, OFP harvesting can be an economic entity for CW operators, providing social and economic benefits, such as the improvement of system landscapes and a better habitat quality. Some authors have reported that polyculture systems enhanced the CW resistance to environmental stress and disease (Karathanasis, Potter and Coyne 2003; Prata et al., 2013).

7.1.2.2 Common Ornamental Plants Used in CWs

Limited quantities of ornamental flowering plants (OFP) have been used in CWs. These types of plants are typical of subtropical and tropical regions. Our survey showed that the four most frequently used genera are, in order of most to least frequently used: Canna spp, Iris spp, Heliconia spp, Zantedeschia spp (Table 4). Species of the Canna genus are used in all continents, with Asia using them the most frequently. The Iris genus is also used in Asia, along with Europe and North America. Species of the Heliconia genus are commonly used in Asia and America, including Mexico, Central and South America. While Zantedeschia is most frequently used in Mexico (a country in North America), they are found with less frequency in Europe, Africa, and Central and South America. The use of OFP in CWs is most popular in tropical and subtropical regions, due to the warm temperatures and the extensive sunlight hours. Such environmental features stimulate a richer biodiversity than in other regions.

Table 4. Four most commonly genera plants used in CWs around the globe, identified during the 87 survey studies in 21 countries, grouped by continents.

|  | America |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Asia | Europe | North America |  | Central and South America | Africa | Total |
|  |  |  | USA | Mexico |  |  |  |
| Canna | 22 | 4 | 5 | 4 | 2 | 2 | 39 |
| Iris | 5 | 5 | 4 | 2 | 2 |  | 18 |
| Heliconia | 4 |  |  | 4 | 4 |  | 12 |
| Zantedeschia |  | 2 | 1 | 13 | 3 | 1 | 20 |

7) Original line no.: 298

Line no., after correction: 511

Reviewer comments: Table 1. Does not show used substrates from industrial waste or everyday waste such as PET, tires, etc. Please add recent information on these and discuss them in the document.

Corrections are made by Authors: We have added recent information on the list of substrates which marked with green colour.

Table 5. Substrates typically chosen for management of wastewater in CW.

| Kinds of Substrates | Source |
| --- | --- |
| Natural Material |  |
| Sand | Saeed and Sun, 2013 |
| Gravel | Calheiros et al., 2008 |
| Clay | Calheiros et al., 2008 |
| Calcite | Ann et al.,1999 |
| Marble | Arias et al., 2001 |
| Vermiculite | Arias et al., 2001 |
| Bentonite | Xu et al., 2006 |
| Dolomite | Ann et al.,1999 |
| Limestone | Tao and Wang, 2009 |
| Shell | Seo et al., 2005 |
| Shale | Saeed and Sun., 2012 |
| Peat | Saeed and Sun., 2012 |
| Wollastonite | Brooks et al., 2000 |
| Maerl | Saeed and Sun., 2012 |
| Zeolite | Bruch et al., 2011 |
| Industrial by-product |  |
| Slag | Cui et al., 2010 |
| Fly ash | Xu et al., 2006 |
| Coal cinder | Ren et al., 2007 |
| Alum sludge | Babatunde et al., 2010 |
| Hollow brick crumbs | Ren et al.,2007 |
| Moleanos limestone | Mateus et al., 2012 |
| Wollastonite tailings | Hill et al.,1997 |
| Oil palm shell | Chong et al., 2013 |
| Artiﬁcial products |  |
| Activated carbon | Ren et al., 2007 |
| Light weight aggregates | Saeed and Sun., 2012 |
| Compost | Saeed and Sun., 2012 |
| Calcium silicate hydrate | Li et al., 2011a |
| Ceramsite | Li et al., 2011a |
| Others |  |
| Alum sludge | Liu et al., 2016 |
| Apatite material | Harouiya et al., 2011 |
| Biochar | Zhou et al., 2018b |
| Bauxite | Cherukumilli et al., 2017 |
| Construction wastes | Shi et al., 2017 |
| Tire chips | Reddy et al., 2017 |
| Polyethylene terephthalate (PET) | Tatoulis et al., 2017 |
| Filtralite | Albuquerque et al., 2009 |
| Oyster shell | Park and Polprasert, 2008 |
| PHBV and PLA blend Wood mulch | Yang et al., 2018 Saeed and Sun, 2011a |
| Rice straw | Cao et al., 2016 |

8) Original line no.: 668-708

Line no., after correction: 936-981

Reviewer comments: Section 10. I believe that it should be called the direction of future research and challenges in terms of the sustainability of CWs. In this, we will speak specifically about what are the lines of action to follow in future research, what are the challenges that technology faces for its greater acceptance, mainly in regions of the world where its use is required, although it is touched the topic the document shows more areas of opportunity in the topic.

Corrections are made by Authors: We have already corrected the section according to your comments which marked with green colour.

# 10. Direction for future research, challenges and overcomes

It has been broadly acknowledged that after decades of research as well as application, CWs are an effective management system for different wastewaters. The present analysis demonstrates that improvements in the design as well as operation of CWs, have significantly enhanced the efficiencies in the elimination of pollutants, and have enhanced the renewable implementation of this management process. For instance, a reasonable modification of the hydraulic design, mode of operation, contaminant loading rate, along with likely choice of plants including substrates may attain the outstanding efficiency in CWs for curing max intensity of wastewater or during winter.

Direction for future research

Direction for future research on the design along with operation of CWs for treating wastewater are provided in Table 6.

Table 6. Direction for future research on design and operation of CWs.

| Parameter | Design Criteria |  |
| --- | --- | --- |
|  | FWS CWs | SSF CWs |
| Bed size (m2) | Larger if available | <2500 |
| Length to width ratio | 3:1–5:1 | <3:1 |
| Water depth (m) | 0.3–0.5 | 0.4–1.6 |
| Hydraulic slope (%) | <0.5 | 0.5–1 |
| Hydraulic loading rate (m/day) | <0.1 | <0.5 |
| Hydraulic retention time (day) | 5–30 | 2–5 |
| Media | Natural media and industrial by-product preferred, porosity 0.3–0.5, particle size <20 mm (50–200 mm for the inﬂow and outﬂow) |  |
| Vegetation | Native species preferred, plant density 80% coverage |  |

Challenges and overcome

Provided the progressively stringent water safety requirements for wastewater management and water recycle globally, moreover, CWs also has few challenges, as well as more study and innovation work are needed. To sum it up (Figure 3):

Figure 3. Summary of Assumptions for achieving sustainable development of CWs.

- Choice of plants and substrates shows that wetland phytoplankton including substrates are already key to the effective elimination of pollutants from wastewater in CW. Careful choice of macrophyte varieties should be given much emphasis (such as broad formation of vegetation, abundant) delivery of O2 and C substances, greater absorption of toxins, particularly evolving toxins, like heavy metals and medical devices, strong contaminant overload resistance) implemented in CWs in warm and cool environments for the management of wastewater, complex and expensive assessment of varieties and seasonal variations is further required. In conjunction, certain non - traditional wetland materials should be produced and utilized for CWs (manufacturing by product, farm garbage, and so on.) that has excellent adsorption potential and is advantageous for removing techniques.

- The analysis of design and operational parameters reveals that the optimum efficiency of the treatment depends fundamentally on ecological, hydraulic and operational situations. Consequently, in future research, improving these situations requires comprehensive and detailed examination. In addition, attention should also be given to researching the main pathway and technique relating to stronger contaminant elimination.

- Considering the ongoing progress of study and functional implementation in conventional CWs, in future experiments, innovative techniques and approaches for improving wastewater implemented in CWs are urgently needed to augment the effective water freshness. Such techniques and approaches usually includes: artificial ventilation, tidal activity, phase feeding, extrinsic incorporation of C, microorganisms increase, distribution of different plants, mixture of different substrates, CWs and hybrid CWs, respectively.

- Vitamins and minerals as well as other contaminants integrated by wetland plants are recorded to be able to discharge into water whenever plants die and decompose however during snowy weather, that may contribute to the low elimination efficiency in CWs. Experiment and modernization on effective techniques for plant harvesting, and purification and regeneration of plant materials in CWs are therefore important.

9) Original line no.: 709-716

Line no., after correction: 983-1006

Reviewer comments: Finally, the conclusion must describe whether the objective set in the study was met promptly. Regarding references, they should be enriched with more current
references

Corrections are made by Authors: We have given conclusion in the following. We have already enriched my review article with more current references.

11. Conclusions

This review-based study illustrates that the factors for CW design and operation such as plant selection, substrate selection, water depth, loading rate, hydraulic retention time, and feeding mode are crucial to achieve the sustainable treatment performance. Wetland macrophytes and substrates represent two factors that influence the efficiency of pollutant removal in CWs. More attention should be given to appropriate plant species selection for CWs. An intensive evaluation of differences between species and season is also needed. In addition, some non-conventional wetland media, characterized by high sorption capacity, should be studied and used for CWs. Moreover, the review of the design and operating parameters shows that the optimal treatment performance is vitally dependent on environmental, hydraulic and operating conditions. Therefore, understanding how to manage and optimize these conditions warrants more investigation. Additional research on the critical pathways and mechanisms corresponding to higher pollutant removal should be taken into consideration. The review of design and operation parameters (plant and substrate selection, and hydraulic conditions) shows that the optimal treatment performance is crucially dependent on hydraulic, environmental, and operating conditions. Therefore, if an optimization of the design and management of these systems wants to be accomplished, further studies on the aspects above mentioned would be needed. For the sake of clarification, it is worth mentioning that as well as studies on design and operation parameters, additional research on maintenance processes and new strategies and technologies is necessary for sustainable CW systems and water quality improvement. Taking into account the efficient and sustainable implementation of full-scale CWs, future studies should focus on a comprehensive assessment of plants and substrates in field trials under real conditions, optimization of environmental and operational parameters, exploration of novel enhancement technologies and maintenance strategies. New studies will provide information that will increase the successful application and sustainability of CWs.