Response to Reviewer 1 Comments

I would like to thank Reviewer 1 for constructive feedback. Please find below my response to the comments.

General comments:

- In section 1 the author describes different baculovirus backbones and transfer-vectors with a strong focus on homologous recombination-based systems. The author only briefly mentions that bacmid based systems are available as well, but industrial use might be limited due to their lower genetic stability (are their additional references available). Nevertheless, readers might be interested in these vectors as well because there are interesting systems available allowing for the co-expression of several different genes from one viral backbone or performing virus based cellular engineering.

Response: This review paper specifically aims to discuss innovations towards industrial scale (i.e production at 1000L or greater) recombinant antigen production, hence the strong focus on homologous recombination as the genetic stability of the recombinant baculovirus is critically important to support scale-up of virus banks. I agree that readers may have an interest in the Bacmid based systems, esp. due to the ease of handling and good performance in the laboratory benchmark study (Stolt-Bergner et al., 2018) and have added an additional reference in the text that discuss the various other systems. Also, please note that the review already includes some of these features when describing the Oxford Expression System flashBAC vectors that are in part based on the Bacmid technology. Personally, I do not have experience with the stability of recombinant baculoviruses made using these vectors and caution should be given to the possibility of instability during scale-up as we did experience with the Bacmid vectors consistent with what has been described in the literature.

I added the following text in the introduction:

“Here we focus on the of a recombinant baculovirus through homologous recombination as this method has shown to be useful for industrial scale production”. …….. “The other broadly used laboratory method to generate recombinant baculovirus through site-specific transposition of Tn7 transposon into a bacmid containing the baculoviral genome (Bac-to-Bac Expression System [Invitrogen, Inc]) has limited industrial use due to its inherent instability during scale-up.”

Reference added in addition to the review by Van Oers et al.:

Fabre, M. L.; Arrías, P. N.; Masson, T.; Pidre, M. L.; Romanowski, V. Baculovirus-Derived Vectors for Immunization and Therapeutic Applications. Emerging and Reemerging Viral Pathogens, 2020, 197–224.

- In section 4 (Downstream processing) the author describes different cell line / virus engineering efforts to reduce process related impurities but is not mentioning any technological possibilities for improvement in the fields of chromatography etc. Maybe the author can extend that section because downstream processing plays a crucial role especially when it comes to the production of virus-like particles.

Response: Section 4 was revised to address the reviewer’s comments. Figure 3 was replaced by the following text

“An example of a universal purification process has been described for recombinant hemagglutinin produced insect cells [6]. Briefly, infected cells (containing rHA) are removed from the protein production bioreactor(s) and separated from the culture media by centrifugation. The cell pellet containing insect cell membranes is solubilized in buffer containing a non-ionic detergent and the extract is subsequently clarified using depth filtration. The rHA-containing extract is subsequently applied to an ion-exchange column. The buffer pH is selected such that rHA proteins will bind to the cation exchange column. DNA contaminants will bind to the anion column. rHA is eluted and then applied to a hydrophobic interaction column where it binds strongly whilst most protein contaminants flow through. The rHA containing eluate is then applied to a Q-membrane to remove residual DNA and finally ultrafiltration is used to formulate the protein in its final buffer.”

An additional paragraph was inserted to describe the purification of a product from the media:

“In the event the protein or product of interest is secreted into the media key will be to separate the contaminants present in the media from the product of interest. Generally, the first step in such a purification scheme includes concentration of the media to reduce processing volume. Subsequently the product of interest can be captured using affinity chromatography and/or ion-exchange chromatography. The main challenge when purifying a secreted product is presented by the recombinant baculovirus present in the media that often has similar biochemical characteristics as the product of interest. This could possibly be addressed through genetic engineering as described below.”

- I would ask the author to re-work the figures because line-art is often not appropriate.

Response:  I decided to remove Figure 1 and 3 and replace this by further explanation in the text.

Specific comments:

- Line 25: The author mentions that insect cells can perform glycosylation. Although this is true pre se patterns greatly differ from mammalian structures. This can lead to the fact the glycoengineering might be necessary to express the antigens correctly.

Response:  The following language was included to address the reviewer’s comment.

“The glycosylation profile is different from mammalian cells and glycoengineering has been successfully used to produce complex sialylated glycans and to eliminate α3 fucosylation either by generating transgenic cell lines or by using modified baculovirus vectors [Palomares et al., 2021].”

- Line 33: Please rephrase: “….in size- or AcNPV ()- that can easily…) because it is misleading.

Response:  The sentence was rephrased as follows to avoid any confusion:

“Baculoviruses (such as AcNPV [Autographa californica Nuclear Polyhedrosis Virus]) have a double-stranded DNA genome of approximately 130 kb pairs in size”

- Line 42: What is the essential gene ORF1629 for?

Response: The following test was included to explain the function of ORF 1629

“ORF1629 encodes a phosphoprotein that is a key component of the nucleocapsid and generally accepted as essential to viral replication [8].”

- Line 142: Readers might be interested in the names of the commercial transfer vectors.

Response: The website of each supplier is provided in Table 1 so that interested reader can easily find what they are looking for. Suppliers often have multiple vectors and this way the Table remains simple and readable.

- Line 221: Please rephrase: “Insect cells grown are generally…..”

Response: Thank you. The word “grown” was deleted from the sentence.

- Line 231: MOI of one for protein production. Is this always the case?

Response: No - The language includes the word “generally” and the next sentence indicated that conditions may be “unique” for each protein and therefore it is recommended to determine optimal conditions”

- Line 235: The figure shows a 50 fold dilution of the virus for infecting the main culture. Du to the fact the MOI is based on the cell count this is misleading.

Response: Here we specifically use “dilution” as conditions for industrial scale production need to be simple and reproducible. Hence, we determined robust growth conditions for both cells and virus banks that could be easily used and result in consistent performance.

- Line 288: Pluronic F-68 is currently mostly marketed under the name Kolliphor P188.

Response: The sentence was adjusted as follows to address this comment:

The non-ionic detergent Kolliphor ® P188 (also known as Lutrol® F68, Pluronic® F-68, Synperonic® F68, Poloxamer 188)

- Line 304: Table 2 (Line 1 is not readable).

Response: Working with the Editor to address this comment as the original Table had the text presented vertical, which was lost in the conversion process

- Line 317: NaB or NaBu?

Response: Thank you. This was a typo – it should be NaB

- Liner 339: The is no graphic for extraction in figure 3.

Response: The Figure has been replaced by Text that includes the extraction step.

Submission Date

29 October 2021

Date of this review

08 Nov 2021 14:54:21