Response to Reviewer 1

Dear Reviewer 1

Comments and Suggestions for Authors

The study Utility of real-time three-dimensional echocardiography for the assessment of right ventricular morphology and function in large animal models has evaluated the relationship between RV function obtained by RT3DE and RV pressure-volume loops and mainly concluded that RT3DE could detect the changes in catheterization-derived RV volume with strong agreement. This original work may provide an interesting novel method for noninvasive assessment of right ventricular function. However, the manuscipt is still lacking in the designing of protocol, the presentation of the experimental results and the justification of the conclusions, and further revisions are required.

Response: We wish to express our strong appreciation to the Reviewer for their insightful comments on our paper. We feel the comments have helped us significantly improve the paper. We hope that the revised paper meets your approval and will be more suitable for publication in the Journal of Clinical Medicine as Original Research. Please see the following point-by-point responses for details.

Materials and Methods:

- (Line17-19) RV pressure-volume loop-derived hemodynamic measurements and echocardiography including teo-dimensional (should be: two-dimensional) speckle-tracking echocardiography...

Response: Thank you very much for your comment, and we apologize the wrong letter. We have corrected the words.

Line 18-20: RV pressure-volume loop-derived hemodynamic measurements and echocardiography including two-dimensional speckle-tracking echocardiography and RT3DE were performed in each study protocol.

- (Line 100, 132+) There were no transducer parameters marked in the echocardiographic part, so it was unclear whether it is suitable for applying in dog

Example:

Two‐dimensional echocardiography (2DE) was carried out in both groups using an iE33 echocardiographic system (Philips Medical Systems, Andover, MA) with an S5-1 transducer (1-5 MHz) Real time three-dimensional echocardiography (RT3DE) was performed using iE33 echocardiographic system equipped with an X3-1 transducer (1-3 MHz) in both gruops.

Response: Thank you very much for your comment. We have added the information about transducer used in this study according to the Reviewer’s comment. Additionally, to avoid the duplication, we have deleted the sentences about the echocardiography equipment in Line 133-134.

Line 97-102: After a 10-min stabilization period, pressure-volume loop measurement (Σ5 DF Plus; Taisho Biomed Instruments Co., Ltd., Osaka, Japan), cardiac output measurements obtained using the thermodilution technique (LabChart Pro; ADInstruments, Aichi, Japan), and echocardiography (Vivid E95 Ultra Edition with an 4Vc-D [1.7–3.3 MHz] or 6Vc-D [3.1–6.2 MHz] transducer; GE Healthcare, Tokyo, Japan) were performed to obtain base-line data.

- (Line 102) 5 and 10 µg/kg/min dobutamine for 10 min....Were these two doses of dobutamine used sequentially? Was there a washout period in between? It was not clearly described in the manuscript.

Response: Thank you very much for your comment. In this study, there were no washout period. We have clarified the study protocol as follows:

Line 104: These two doses of dobutamine were infused sequentially without washout period.

- (Line 105-106) Acute volume loading was performed by infusing lactated Ringer's solution at a rate of 150 mL/kg/h for 30 min.... It was unclear whether there was a washout period between the injection of lactated Ringer's solution and the injection of dobutamine previously. If not, dobutamine might affect morphology and functional variables in Table 2.

Response: Thank you very much for your comment. As noted in Line 105-106, a 10-min washout period with lactated Ringer’s solution was put before the baseline measurements for acute volume loading.

- (Line 108-109) The same examinations as those at baseline were performed 15 and 30 min after acute intravenous infusion and furosemide administration. What is the basis for determining the two time points of 15 and 30 minutes? The action time of furosemide in the human body can be maintained for 2 hours. Should multi-group time-effect relationships be considered in the study protocol and extended the observation period to 2 hours?

Response: Thank you very much for your comment. As you mentioned, it is possible that the more substantial changes in volume loading conditions could be confirmed by observing for a longer period. However, because there were significant decreases in RV volume based on right heart catheterization 15 and 30 min after furosemide administration, we considered that the purpose of this study to change the volume load was fully achieved.

- (Line 133) Conventional and Doppler echocardiographic examinations and measurements.... Here, “conventioal”could be change into “2D”.

Response: Thank you very much for your advice. We have changed the words according to the Reviewer’s comment.

Line 134-135: 2D and Doppler echocardiographic examinations and measurements were per-formed by the same observer (YY).

- (Line 140) end-diastolic RV internal dimension (RVIDd)....RVIDd could be measured in several sub-segments (basal, middle...) in RV, which typical segment was measured in the study? Please describe them in detail.

Example:

Transverse diameter of right ventricular basal segment (RVD1) and middle segment (RVD2) were collected for analysis.

Response: Thank you very much for your advice. In this study, RVIDd was measured as the largest transverse diameter at the middle RV. We have clarified the definition of RVIDd as follows.

Line 141-142: The RVIDd was measured as the largest transverse diameter at the middle RV.

- (Line 163) RV quantification (4D Auto RVQ; GE Healthcare) ....The software information was not detailed enough, please add the version number.

Example: TomTec 4D LV analysis (4.6.0.411, TomTec Imaging Systems GMBH, Germany)...

Response: Thank you very much for your comment. The application used for the RV quantification were equipped with the echocardiographic system used in this study. Therefore, there is no version number of the application.

- (Line 175-176) ...generated by the right ventricular quantification software... please change “quantification software”into “4D Auto RVQ software” to be more specific.

Response: Thank you very much for your advice. We have changed the words according to the Reviewer’s comment.

Line 177-178: Figure 1. A representative image of the right ventricular volume generated by the 4D Auto RVQ software.

- (Line 175-176) The coefficient of variation (CV) and intra- or interobserver correlation coefficients (ICC) were calculated to evaluate the intra- and interobserver measurement variability. Low measurement variability was defined as CV < 10.0 and ICC > 0.7.

Could add: If CV ≥ 10.0 and ICC ≤ 0.7, the data should be judged by a third blinded independent observer.

Response: Thank you very much for your advice. We have added the sentence according to the Reviewer’s comment.

Line 187-188: If CV ≥ 10.0 and ICC ≤ 0.7, the data should be judged by a third blinded independent observer.

- (Line 200-201) ....correlations between RV volume obtained by right heart catheterization and RV morphological variables obtained by 2DE (RV area and RVIDd).....RT3DE has RVEDV3D and RVESV3D could be evaluated RT3DE, why not use these for analysis ?

Response: Thank you very much for your suggestion. In this study, 2DE variables (RV area and RVIDd) were measured to compare the clinical utility between 2DE and RT3DE.

Results

- (Line 209) In this study, one dog skipped the study protocol of dobutamine infusion... Please add some detailed information, like carbon dioxide, percutaneous saturation oxygen, and blood pressure level at the time of the dog skipped the study.

Response: Thank you very much for your comment. Carbon dioxide, percutaneous saturation oxygen, and blood pressure level of the dog that skipped the study protocol of dobutamine infusion were within the reference intervals at the time of the dog skipped the study. We have added the detail information about the dog that skipped the study protocol of dobutamine infusion.

Line 217-220: Although carbon dioxide, percutaneous saturation oxygen, and blood pressure level were within the expected ranges noted above, one dog skipped the study protocol of dobutamine infusion because frequent ventricular premature complexes associated with dobutamine infusion complicated the hemodynamic measurements.

- (Line 211-213) The end-tidal partial pressure of carbon dioxide, percutaneous oxygen saturation, and blood pressure measurements were within the expected ranges in healthy dogs throughout the experimental procedures. What was the expected range? Values for carbon dioxide, percutaneous oxygen saturation, and blood pressure should be described.

Response: Thank you very much for your advice. We have added the information about the expected ranges of carbon dioxide, percutaneous oxygen saturation, and blood pressure measurements.

Line 214-217: Throughout the experimental procedures, all healthy dogs showed the expected ranges of the end-tidal partial pressure of carbon dioxide (35–45 mmHg), percutaneous oxygen saturation (98–100%), and blood pressure measurements (mean systemic arterial pressure: > 60 mmHg).

- (Line 249-250) Although RV volume was measured using right heart catheterization and RT3DE, no substantial changes in RVEDA, RVESA, and RVIDd were noted.... RVIDd could be detailed evaluated in segments (see Materials and Methods:7)

Response: Thank you very much for your comment. According to the Reviewer’s comment, we have clarified the definition of RVIDd. Unfortunately, basal RV internal dimension has not been measured in this study. If Reviewer considers that basal RV internal dimension is also needed, we will include the results immediately.

- (Line 266) a.The value is significantly different compared with baseline. ...The legend has two a, please change it into a proper word.

Response: Thank you very much for your comment, and we apologize that the same content was written twice. We have modified the sentence as follows.

Line 272: a The value is significantly different compared with baseline.

- (Line 271-294, part 3.3) Now that the regression coefficient was listed, the regression equation could be listed simultaneously.

Response: Thank you very much for your advice. We have added the regression equations of Ees and Ees/Ea as follows.

Line 280-284: After adjusting for confounding factors, SV3D, RVESA, RV FAC, RV s’, and RV-SrL6seg were included in the multivariate model to estimate Ees, and RV s’ and RV-SrL6seg remained significant in the multivariate model (adjusted r2 = 0.66, p < 0.001): Ees = 0.07 × (RV s’ [cm/s]) + 0.56 × (RV-SrL6seg [%/s]).

Line 288-292: After adjusting for confounding factors, EF3D, SV3D/RVESV3D, RV s’, and RV-SL3seg were in-cluded in the multivariate model to estimate Ees/Ea, and RV s’, SV3D/RVESV3D, and RV-SL3seg remained significant in the multivariate model (adjusted r2 = 0.34, p < 0.001): Ees/Ea = 0.03 × (RV s’ [cm/s]) + 0.30 × (SV3D/RVESV3D) + (-0.14) × (RV-SL3seg [%]).

- (Line 304-305) All variables had proportional errors (maximal and RVESV; EF, p < 0.001; SV, p = 0.014). What didit mean? Might be change into “maximal are RVESV, p < 0.001; EF, p < 0.001;...”

Response: Thank you very much for your comment, and we apologize the wrong letter. We have corrected the words as follows.

Line 312-313: All variables had proportional errors (RVEDV, RVESV, and EF, p < 0.001; SV, p = 0.014).

- (Line 314-315，Fig 2) (c) ejection fraction (EF); (d) stroke volume (SV) should be “(c) stroke volume (SV); (d) ejection fraction (EF)”

Response: Thank you very much for your comment. We have modified the sentence according to the Reviewer’s comment.

Line 320-323: Correlations between the right ventricular morphology and function measured by right heart catheterization and real-time three-dimensional echocardiography: (a) end-diastolic right ven-tricular volume (RVEDV); (b) end-systolic right ventricular volume (RVESV); (c) stroke volume (SV); (d) ejection fraction (EF).

- (Line 315-316, Fig 2) Green, orange, and blue dots show the measurement values ​​of baseline, dobutamine infusion, and acute volume load, respectively....dobutamine infusion was in two doses (5 µg/kg/min, 10 µg/kg/min), acute volume overload had15 min and 30 min data, and after Furosemide infusion there were 15 min and 30 min data, all of them could be represented respectively by dots with different colors/shapes.

Response: Thank you very much for your advice. We have modified the dots according to the Reviewer’s comment. Please see the Figure 2 for the details.

Line 323-327: Black round dots show the measurements of baseline. Orange round and triangular dots the measurements of 5 and 10 µg/kg/min dobutamine infusions, respectively. Blue round and tri-angular dots the measurements of 15 and 30 min after acute volume load, respectively. Green round and triangular dots the measurements of 15 and 30 min after furosemide administration, respectively.

- (Line 321, Fig 3) (c) ejection fraction (EF); (d) stroke volume (SV)..should be “(c) stroke volume (SV); (d) ejection fraction (EF)”.

Response: Thank you very much for your comment. We have modified the sentence according to the Reviewer’s comment.

Line 329-332: Agreements between the right ventricular morphology and function measured by right heart catheterization and real-time three-dimensional echocardiography demonstrated on Bland-Altman plots: (a) end-diastolic right ventricular volume (RVEDV); (b) end-systolic right ventricular volume (RVESV); (c) stroke volume (SV); (d) ejection fraction (EF).

- (Line 322-234, Fig 3) Blue, orange, and green dots show the measurement values of baseline, dobutamine infusion, and acute volume load, respectively. (Similar topiont8) dobutamine infusion was in two doses (5 µg/kg/min, 10 µg/kg/min), acute volume overload had 15 min and 30 min data, and after Furosemide infusion there were 15 min and 30 min data, all of them could be represented respectively by dots with different colors/shapes.

Response: Thank you very much for your advice. We have modified the dots according to the Reviewer’s comment. Please see the Figure 3 for the details.

Line 333-337: Black round dots show the measurements of baseline. Orange round and triangular dots the measurements of 5 and 10 µg/kg/min dobutamine infusions, respectively. Blue round and triangular dots the measurements of 15 and 30 min after acute volume load, respectively. Green round and triangular dots the measurements of 15 and 30 min after furosemide administration, respectively.

- Only using multiple regression analyses to prove that "RT3DE could detect the changes in catheterization-derived RV volume with strong agreement" is not sufficient, further diagnostic experiments should be designed, ROC curves should be made, and AUC should be reported. This paper lacked such experimental design, so the credibility of the conclusions is questionable.

Response: Thank you very much for your comment. As you mentioned, further diagnostic experiments including receiver operating characteristic curve to detect the changes in RV volume are needed to validate the clinical utility of real-time three-dimensional echocardiography. We have added the following sentences into the limitation section.

Line 452-454: Further diagnostic experiments including receiver operating characteristic curve to detect the abnormal RV morphology and function are needed to validate the diagnostic utility of RT3DE in the future.

Discussion

- (Line 365-366) .......evaluate only the RV inflow region, additional views that could evaluate the other regions would be needed....The echocardiography foucus more in RV inflow region, the situation of RV outflow region remained unclear, saying “variables showed only slight changes along the acute volume load (Line 361-362)” was no adjusted. Consider changing to "2DE variables of RV inflow region showed only slight changes..."

Response: Thank you very much for your advice. We have changed the sentence according to the Reviewer’s comment.

Line 373-375: Although the RV area and RVIDd obtained by 2DE had similar tendencies to the catheterization-derived RV volume, 2DE variables of RV inflow region showed only slight changes along the acute volume load.

- (Line 372) However, dobutamine infusion and acute volume load caused a dissociation between right heart catheterization and RT3DE in several measurements.....Since "tracing the RV endomyocardial border might be difficult" (Line 376-377), whether applying Repeated measurements to reduce measurement error?

Response: Thank you very much for your comment. As you mentioned, repeated measurements might minimize the measurement error for the evaluation of RV volume using real-time three-dimensional echocardiography. We have added the following sentences into the discussion.

Line 391-393: Additionally, a single measurement by the multi-beat method for the statistical analyses might also influenced our results. Repeated measurements might reduce the measure-ment error.

- The sample size n=10 was too limited. In order to exclude the influence of anesthesia itself on RV parameters, a control group without giving dobutamine infusion, acute volume overload, and furosemide infusion.

Response: Thank you very much for your comment. As you mentioned, because there was no control group, this study could not rule out the influence of anesthesia on RV morphology and function variables. We have added some sentences into the limitation.

Line 436-438: Furthermore, there was no control group without giving dobutamine infusion, acute volume overload, and furosemide administration. Anesthesia might influence the results of RV morphology and function.