Type of the Paper (Article, Review, Communication, etc.)

Correlation analysis between roadway networks and economic ranking. Case study: municipalities and departments of Colombia

Carlos Felipe Urazan Bonells 1, Maria Alejandra Caicedo Londoño 2*, and Hugo Alexander Rondón Quintana 3

Citation: Lastname, F.; Lastname, F.; Lastname, F. Title. Infrastructures 2022, 7, x. https://doi.org/10.3390/xxxxx Academic Editor: Firstname Lastname Received: date Accepted: date Published: date Publisher’s Note: MDPI stays neutral about jurisdictional claims in published maps and institutional affiliations. Copyright: © 2022 by the authors. Submitted for possible open access publication under the terms and conditions of the Creative Commons Attribution (CC BY) license (https://creativecommons.org/licenses/by/4.0/).

1	Universidad de La Salle, Bogotá, Colombia; caurazan@unisalle.edu.co

2	Universidad de La Salle, Bogotá, Colombia; macaicedo@unisalle.edu.co

3	Universidad Distrital Francisco José de Caldas, Bogotá, Colombia; harondonq@udistrital.edu.co

*	Correspondence: macaicedo@unisalle.edu.co; Tel.: +57-3178896874 MACL

Abstract: Abstract: It is generally assumed there is a statistically valid correlation between the length of a roadway network, and others like its classification and/or average travel speed, with economic indicators such as Gross Domestic Product (GDP) and the Municipal Relative Weight (MRW), considering that the roadway network and transport development generates economic development in a region. This study reports the results of correlating several variables which are economic indicators of roadway networks, both at a municipal and departmental level in Colombia; and it was concluded that at the level of municipalities there is no valid correlation between MRW as a dependent variable, the average travel speed, and the sum of the length in kilometers of roadways that connect villages as independent variables. There was neither a correlation resulting between the MRW as an independent variable nor the traveling distance and time for each municipality concerning the capital city of each respective department. Finally, it was found that the department agribusiness GDP is associated with the length of the tertiary roadway network and with the primary network with an R2 of 0.7. This study concludes that activities in rural zones are the ones that generate the greatest impact on roadway investment within a region.

Keywords: Gross Domestic Product, Tertiary Roadway Network, Municipal Relative Weight.

1. Introduction

Colombia is a country that has an urgent need to revitalize rural activity, making it more productive. Adequate transportation of agricultural products is essential for this development, and that is why it´s necessary to deepen the study of the variables by which the economic investment in improving rural roads is decided.

When studying the relationship between the economic development of a country´s regions and the condition quality of the road network, it is found that several studies are showing the existence of a relationship between roadway networks and the economic condition of a region or territory, especially in rural areas. However, few studies present a relationship between the conditions of a roadway network with a macroeconomic variable such as Gross Domestic Product. (GDP). Urazan et al. (2017) [1] concluded that for a sample of countries that are representative of Latin America and the Caribbean, there was a statistically valid correlation between the length of a primary level roadway network of the analyzed countries with their GDP. Contrary to the above, the independent variables, the second-level roadway network, and the paved network did not statistically correlate with GDP.

Jacoby (2000) [2] described the important role of roadways in rural development. In the case of Nepal, he concludes that providing extensive roadway connections to agricultural markets leads to important benefits to poor homes located in the area. However, he clarifies that these benefits are not enough to be able to conclude the achievement of reducing poverty or inequality.

Escobal and Ponce (2002) [3], mentioned that the relationship between poverty reduction and the availability of rural infrastructure has broadly been discussed since the decade of 1990. According to them, studies conducted in South East Asia demonstrate the existence of a relationship between massive investment in rural roadways and posterior economic growth, as well as the reduction of rural poverty. Jalan y Ravallion (2002) [4] concluded that economic progress in rural areas not only depends on the development of roadway infrastructure, it also depends on other basic infrastructures such as electric power.

Meerzman and Nazemzadeh (2017) [5] established that in the case of Europe (Belgium), the development of transport infrastructure as an economical improvement driver is not always constant. Important differences may be present between the regions of one same country, especially given the presence or absence of other elements which favor this economic growth.

Howe and Richards (2019) [6] reported that in the South Sahara area there is no clarity in the relationship between investment in rural roadways (associated with lower transport costs) and greater agricultural productivity which implies better economic conditions for the community. This paper also studies cases in India and concludes that improvements in agricultural production occurred through technological improvements, and do not have a strong relationship to roadway improvements for accessibility. In the case of Japan, they mentioned, that there have been important improvements in agricultural production with few roadway kilometers to travel and that economic growth in rural areas is not necessarily caused by agricultural activities. In other words, there are other types of activities that contribute to economic activity and therefore, to GDP.

Asher and Novosad (2020) [7] concluded that, at a worldwide level, close to 1000 million people live in rural areas with no access to a national roadway network connected through paved roads. In the case of India, several years after the construction of rural access roadways, their main effect has been facilitating the arrival of agricultural workers, however, this did not cause an improvement in economical income or greater productivity. Furthermore, even with better roadway connections to markets, remote rural areas may continue to lack better economic opportunities.

Nautiyal and Sharma (2021) [8] proposed several criteria for conducting roadway maintenance. The main and most critical criterion is the improvement of the asphaltic layer, which affects average travel speed. Pavement condition was followed by an analysis of the area of influence's social and economic activity, as well as its connectivity to other communities. According to them, the rural roadway network has a significant effect on the development of a nation.

In an analysis of economic growth in Colombia, Cortés and De la Peña (2019) [9] stated that the increase in transport infrastructure is essential to increase competitiveness within countries that are members of the Pacific Alliance. They score the case of Colombia as lagging in its load transport infrastructure in roadways, railways, and ports; given investment in this sector has been 3.2%, when the recommendation of multinational entities is investing close to 6%. Regarding the quality of infrastructure, Colombia is behind Peru, Chile, and Mexico.

Palei (2015) [10] based his arguments on World Bank statistics to state that one of the main factors that relate to the economic development and competitiveness of a country is the quality of its roadways, railway infrastructure, and airport infrastructure, as well as the electric power supply network.

Bottasso et al. (2021) [11], studied the case of Brazil, and established that the length of a roadway infrastructure between the years 2005-2015 significantly helped to have a greater investment in load transport systems; especially in remote areas and maritime ports.

Idei and Kato (2018) [12] researched the effects of improvement in roadways on rural markets. The authors mentioned that in the state-of-the-art review there is no established relationship between the state of roadways and economic benefit in homes within the roadway's influence area. In the case of Cambodia, they conclude that good condition in roadways implies a greater number of commutes regarding agricultural and livestock products, mainly, those transported by bicycle or motorcycle.

Using multivariable models, Gosh and Dinda (2021) [13] analyzed the relationship between transport infrastructure and economic growth in India (analysis period 1990-2017). The study relates indicators of three transport systems with the per capita GDP in India. In the main conclusion, they mention that air and roadway transport have positive long-term impacts and that railway transport is not important.

In the case of China, Magazzino and Mele (2020) [14] studied 28 regions with data from 1990 to 2017 concluding that transport infrastructure investment differs from region to region and affects the economic growth to the aggregate level.

Working in Pakistan, Batool and Goldmann (2020) [15] studied the relation between private and public investment in transport infrastructure to promote more private investment that results in better values for the GDP.

Rokicki and Stepniak (2018) [16] concluded that the investment in transport infrastructure in Poland between 2004 y 2014 resulted in a better accessibility indicator but is not statistically significant with the growth of regional productivity in urban areas, but negatively correlated with growth in rural areas.

Studying the Wider Economics Impacts WEI, Rothengatter (2017) [17] concluded that the assessment of investment in transport infrastructure on the GDP is only valid in industrialized countries. In another context, Hansen and Johansen (2017) [18] studied the economic impacts of several transport infrastructure projects in Norway, also using the wider economic impacts, but setting as travel variables the commuting and migration flows between regions.

In the case of Colombia, a document of Consejo Nacional de Política Económica y Social (National Council for Economic and Social Policy) (CONPES, 2016) [19] shows in Graph 4, that there has been a trend for greater Government investment in rural roadways within territories with the greater rural population. However, Graph 3 shows that there was no Average investment trend in territories where there is a greater length of the tertiary roadway network. Escobar et al. (2016) [20] mentioned that it is popularly acknowledged that in any region of Colombia, having a good roadway network is related to good economic levels, and increasing quality of life for inhabitants. However, the article does not analyze a correlation between the GDP and the Roadway Network Density Index RNDI.

In summary, there are no frequent studies that analyze a correlation between the quantity and condition of roadway networks of a municipality or department (States or autonomous communities in the case of other countries) with variables that measure the economic level within a territory. Several of the cases found that analyze the relationship between transport infrastructure and economic growth, do not differentiate between urban and rural areas. The greater portion of studies supports the idea of a contribution of rural roadway networks to rural development or territorial progress in the function of roadway conditions. This study contributes to the state-of-the-art, by attempting to correlate travel conditions (time, average speed) and roadway length, with economic development indicators such as departmental Gross Domestic Product (GDP) and Municipal Relative (MRW) for municipalities.

The purpose of the study is to perform a basic statistical analysis of the correlation between data, like a first approach to establish the economic variables to be studied in greater detail to understand the global impact of transport infrastructure in rural areas; contributing to an important way to the knowledge state of in Latin America. There are no frequent studies that analyze a correlation between the quantity and condition of roadway networks of a municipality or department with variables that measure the economic level within a territory. Aiming at this point, this paper contributes to the state-of-the-art, by attempting to correlate travel conditions (time, average speed) and roadway length, with economic development indicators such as departmental Gross Domestic Product and Municipal Relative Weight for municipalities.

The hypothesis put forward in this manuscript is that the extent of a territory´s rural network is correlated on large scale with its economic productivity.

2. Materials and Methods

Hypothesis and Object of Study

This study proposed a hypothesis that stated that a good condition in the roadway network of territory must correlate to a good economic condition within it. Likewise, such correlation would expect the opposite effect if the roadway network is not in good condition. A study published by Urazan et al. (2017) [1] was the foundation that motivated proposing this hypothesis. Given the above, it was thought that this relation could occur if applied on a national level (Colombian case) correlating information at municipal and departmental levels. This study analyzed the following:

- At a municipal scale, a sample of 247 cases, distributed in 5 departments, with records of roadway length through the shortest route between municipalities and each of their rural villages and districts, and the corresponding travel speed (taken in the field with the use of Global Positioning System – GPS equipment, during working day hours)

- At a municipal scale, for the same sample, but with travel times, distance, and average travel speed. On this occasion, between each municipality and the capital city of that department. These records were georeferenced during labor day hours.

- At a departmental level, adding data from 247 municipalities and grouping it in each of the corresponding 5 departments.

- At a departmental level, for 27 departments in the country, using the following information: length of paved roadway network and length of the non-paved roadway network. The length was analyzed separately according to the following conditions: Very Good, Good, Regular, Poor, and Very Poor. Work also included the length of tertiary and primary roadway networks.

The following are variables that indicate economic condition: MRW of each municipality in its department, global GDP of each department, and sector GDP for agribusiness and mining activities. Several regressions were carried out, combining all the independent variables (length of paved and non-paved roadway network) to evaluate if there is a statistically valid correlation (R2 coarse adjustment and/or above 0.7) (Dagnino, 2014) [21], (Pando and San Martín, 2004) [22], (Roy et al. 2020) [23] to define if any of the characteristics of the roadway network explains or correlates to the economic condition of a municipality within its respective department or a department at a national level.

Methodology

This study was initially applied to municipalities, rural villages, and districts that belong to 5 departments in Colombia, out of which 4 of these (Cundinamarca, Meta, Quindio, and Casanare) conducted travels to obtain average speed values, distance, and travel times, using GPS tracking apps. Records obtained were compared to travel times appearing in Google Maps, concluding that there is an average difference of only 5%. This value allows us to attest that the information provided by Google Maps is reliable. In the fifth department's case (Santander) average travel time and distance were recorded, using only information from Google Maps. Fieldwork and its analysis were conducted as the graduation project of a group of Civil Engineering students from Universidad De la Salle (Bogota, Colombia). The results of these projects are described in García and Silva (2017) [24] – Cundinamarca and Casanare; Carrascal and Cuervo, (2019) [25] – Meta; Marín and Fonseca (2021) [26] – Quindío; Insuasti and Pérez (2021) [27] – Santander (Table A1).

Variables (obtained on the field) that were initially analyzed for each department were the following: average travel speed and distance between municipality village or district. Posteriorly, with the average speed of all studied municipalities, an average value was established for each department. Total kilometer distance was also calculated for each municipality to its rural villages and districts, and the total kilometer distance of each municipality was added to obtain an average value per department.

It is important to highlight that the study was only carried out for roadways that connect to the 247 municipalities selected (in the previously cited graduation projects) with their respective rural villages and districts. Given the difficulty of cost and time for carrying out travels, in the Casanare department work was conducted with 18 municipalities, in Cundinamarca with 113 municipalities, and Santander used (Google Maps) with 81 municipalities, including their respective department capital cities.

Posteriorly, the variable that defines the economic position of municipalities within a department is Municipal Relative Weight in the departmental aggregate value (%) (MRW) (Table A1). The data were obtained from the National Administrative Statistics Department - Departamento Administrativo Nacional de Estadística DANE (2020) [28]. The mentioned indicator was selected given that there are no GDP records per Municipality.

The information was analyzed using a parametric statistical study, performing correlations checking initially if the R2 value is close to or upper to 0.7 to validate the relationship between data. In that case, the analysis was continued by checking the standard error and p-value results to complete the parametric study. If the analysis was with multiple independent variables (x1, x2… xn), the adjusted R2 value was verified. The analysis was made in Microsoft Excel software. Software like “R” or similar could also be used.

Based on the study's hypothesis, the length of roadways that connect municipalities to their villages and districts was correlated to the average travel speed obtained by tracking those routes as independent variables, with the MRW as a dependent variable. Out of each correlation verified if adjusted R2 was equal to or greater than 0.7. If the above occurred, verify the coefficient's sign for each of the independent variables to validate its condition of being direct or inverse concerning the dependent variable. If this condition is not met with the adjusted R2, the correlation is discarded. This analysis is carried out to analyze if the correlation of economic condition of municipalities occurs to the length of their rural roadway network and its circling conditions (average travel speed. Greater speed is interpreted as better roadway condition).

It is important to highlight that there is a statistical correlation between a variable and another if when one of them changes value, the other change it too. A way to check if there is an important correlation between variables is the R2 value because is the measure of how far the plot points fall from the regression line. In other words, is calculated with the vertical distance from each point to the trend line. If these distances are small and consistent the R2 value is large and close to 1. On the contrary, the value is small and intimate to 0. A value of R2 equal to or upper to 0.7 means that there is a strong relationship between the data correlated [29], [30], [31}. Remember that the value R2 increase or decrease the level of confidence about the relationship of the hypothetical variables.

If an acceptable R2 is obtained, the validity of the correlation is verified with the analysis of standard error and p-value. The present study only checks that the p-value results are equal to or less than 0.05 and a low value for the standard error in case the R2 is equal to or upper to 0.7. Similar criteria are applied to the R-value value. If the R2 value result is statistically validated, then proceed to confirm with R coefficient analisys.

The statistical analysis or correlation between data in this study is only the first approach to establish the economic variables to be studied in greater detail to understand the global impact of transport infrastructure in rural areas.

A new analysis was conducted in which the arithmetic average of travel time and distance between each municipality was added, however, in this case, the department's capital city was an independent variable that explains the MRW (Table A1). This new analysis is proposed in case there is a good correlation between the economic condition of municipalities and their roadway connection to the capital city of their department, given the capital is its main social and economic connection node.

Then, an aggregate analysis was carried out for municipalities for each of the 5 departments. Correlations between the departmental GDP, average travel speed, and length of kilometer distance values in village connection roadways were analyzed for each department (Table A2).

Posteriorly, we decided to broaden the sample to 27 departments (out of a total of 32 in Colombia) out of which there is information related to roadway conditions is available, through Instituto Nacional de Vías de Colombia (INVIAS) [32]. For this purpose, the analysis used departmental GDP information as a dependent variable (DANE, 2020) [28], and roadway network length according to its condition (paved and non-paved roadways) with the rating of Very Good, Good, Regular, Poor and Very Poor; as an independent variable (Table A3). Data used corresponded to that of 27 INVIAS territories, which correspond to the following departments: Antioquia, Atlántico, Bolívar, Boyacá, Caldas, Caquetá, Casanare, Cauca, Cesar, Chocó, Córdoba, Cundinamarca, Guajira, Huila, Magdalena, Meta, Nariño, Norte de Santander, Putumayo, Quindío, Risaralda, Santander, Sucre, Tolima, Valle and San Andrés.

Lastly, a similar analysis to the above was carried out, however specifying the contribution of GDP for the economic sectors of greatest relevance in a rural scenario: agriculture, livestock, fishing, hunting, and silviculture (presented together as the agribusiness sector), and additionally, added the GDP value of mining (Table A4). Additionally, other correlations were carried out between the agribusiness and mining GDP with the length of primary and tertiary level roadway networks (INVIAS, 2020) [28], for each department (Table A5).

3. Results

3.1. Analysis According to Municipality

The first regressions carried out were for the municipalities selected in each department, using MRW as a dependent variable and average travel speed and the sum of kilometer distance for village connection roadways as independent variables. For the department of Casanare, adjusted R2 for multiple linear regression was (0.062), for Cundinamarca (-0.009), for Meta (-0.064), and Santander (0.36), and Quindío (-0.039). Low R2 values discard a correlation of MRW with these 2 independent variables.

Given the analysis above was carried out with both independent variables and did not result in statistically feasible, the correlation was only carried out with roadway network length that connects villages and districts, proposing the hypothesis that if exist a greater number of kilometers of connections to villages and districts a given municipality has, there is a greater distance to travel, negatively impacting in rural productivity and therefore being related to a lower value MRW. The correlations between roadway network length and MRW were carried out for the following regressions: linear, exponential, potential, and logarithmic. The best R2 value obtained was 0.62 for potential in Casanare. Values for other departments were lower, which rendered the correlation invalid. Other regressions, carried out with average travel speed as the only independent variable, produced R2 values that did not register above 0.32, which discarded that the productive condition of municipalities in their respective department was related to the average travel speed to and from their villages and districts; variable that can be interpreted as the average condition of the roadway infrastructure. (Tables 1 and 2).

Table 1.  R2 value when correlating MRW with the length of tertiary roadway network between villages and districts with their municipal capital.

| Department | Linear | Logarithmic | Exponential | Potential |
| --- | --- | --- | --- | --- |
| Casanare | 0.12 | 0.44 | 0.16 | 0.62 |
| Cundinamarca | 0.003 | 0.007 | 0.003 | 0.016 |
| Meta | 0.007 | 0.023 | 0.003 | 0.04 |
| Santander | 0.37 | 0.31 | 0.04 | 0.24 |
| Quindío | 0.016 | 0.19 | 0.028 | 0.22 |

Table 2.  R2 value correlates MRW and average travel speed between villages and districts with their municipal capital.

| Department | Linear | Logarithmic | Exponential | Potential |
| --- | --- | --- | --- | --- |
| Casanare | 0.01 | 0.05 | 0.03 | 0.002 |
| Cundinamarca | 0.004 | 0.0015 | 0.003 | 0.0006 |
| Meta | 0.029 | 0.011 | 0.02 | 0.019 |

Now, analyzing the previous correlations with R (Pearson correlation coefficient) the highest value obtained was 0.61 (including positive and negative) for the correlation Between MRW and length of tertiary roadway network between villages and districts with their municipal capital for Santander Department. But, the next higher value was 0.36 for the Casanare Department in the correlation of the same variables. The values are presented in Table 3. The validated coefficients sign is positive, for both R and R2. The present analysis confirms the previous conclusions about correlations based on R2 values.

Table 3.  R-value correlates MRW and length of tertiary roadway network and the average travel speed between villages and districts with their municipal capital.

|  | Between MRW and length of tertiary roadway network between villages and districts with their municipal capital | Between MRW and average travel speed between villages and districts with their municipal capital |
| --- | --- | --- |
| Casanare | 0,36 | 0,23 |
| Cundinamarca | -0,06 | -0,07 |
| Meta | -0,09 | -0,17 |
| Santander | 0,61 | 0,21 |
| Quindío | 0,13 | 0,39 |

Posteriorly, an analysis was carried out for the 5 departments (adding values of municipalities of each department). In this case, changes in geographical context (changes from municipality to department) cause the dependent variable for indicating economic condition not to be MRW, but rather Departmental Gross Domestic Product (GDP) (DANE, 2020) [28]. The independent variable was tertiary level roadway network length between municipalities and their villages and districts: Posteriorly, conducting the following regressions: linear, exponential, potential, and logarithmic, for the same dependent and independent variables, none of the resulting R2 values approached a minimum of 0.7, ratifying that there is no valid relationship between the global departmental GDP, roadway length and travel time in a roadway that connects its respective villages and districts (Table 4).

Table 4. R2 value correlates GDP and tertiary roadway network length between villages and districts with its municipal capital.

|  | Linear | Logarithmic | Exponential | Potential |
| --- | --- | --- | --- | --- |
| 5 Departments | 0.0005 | 0.0078 | 0.35 | 0.41 |

The second independent variable analyzed about departmental GDP was the average travel speed on routes between each municipality, its villages, and districts. In this case, the hypothesis is the following: the greater travel speed, the municipality should have a greater development capacity (indicated by MRW) given that travel times are lower. Upon conducting correlations for linear, exponential potential, and logarithmic regressions, the greatest R2 value obtained was (0.25) with linear regression (Table 5). These values discard the correlation between the economic condition of each of the 5 analyzed departments and the average travel speed between several of its municipalities towards and from their respective villages and districts.

Table 5. R2 value when correlating departmental GDP and average travel speed between villages and districts with their municipal capital

|  | Linear | Logarithmic | Exponential | Potential |
| --- | --- | --- | --- | --- |
| 5 Departments | 0.25 | 0.24 | 0.23 | 0.22 |

Up until this moment, the variables of travel between each department's municipality and their connecting villages are not indicators of the economic level within the region. Given the relation of MRW did not result in viable travel between each municipality and its villages and districts, we proceeded to analyze the correlation of MRW by adding travel time and distance between each municipality and the capital city of each department. The correlation result for these 5 independent variables (average travel distance and speed between each municipality and the capital city of each department) resulted in a high value adjusted R2 for Casanare and Quindio (0.73 and 0.97, respectively), however, with very low values for Cundinamarca, Meta, and Santander (0.25, 0.30 and 0.36, respectively). These last numbers do not grant statistical validity to the correlation. Additionally, the signs of coefficients are not equal in the 5 studied departments. This way, we establish that the economic productivity condition of a municipality (about the department it belongs to and measured through MRW) is neither related to roadway network length which interconnects its villages and districts, nor related to the average travel time that takes place in these routes.

Posteriorly, the variable speed was eliminated, given that in the analysis above it was observed that a multicollinearity condition occurred because of the resulting velocity of the direct relationship between travel time and distance. Despite this, adjusted R2 values resulted being low: Casanare (0.32), Cundinamarca (0.17), Meta (0.10), Santander (0.37) y Quindío (0.16). Then, correlations were carried out for all the studied municipalities, without discriminating them by the department. In total, these add up to 247 municipalities. Results also denied a statistical correlation. Using the total kilometer distance to villages and districts and average travel speed in these trajectories, and MRW as a dependent variable, adjusted R2 was only 0.043. The result was similar when correlating MRW only with kilometer distance, just like when using average speed (0.001). Given the above, this analysis discards a correlation between the municipal network connecting to villages and districts and the economic position of each municipality. In a complementary manner, the following regressions were carried out: linear, potential, exponential, and logarithmic, however R2 values varied between 0.003 and 0.12 for the analysis using the length of the tertiary network (Table 6), and between 0.0038 and 0.0074 for the analysis that used average travel speed (Table 7). With the above, it can be ratified that the MRW of municipalities in Colombia does not correlate with roadway network length which connects to its villages and districts, nor with average travel speed in these routes or sections.

Table 6. R2 value upon correlating MRW and the tertiary roadway length between villages and districts with their municipal capital (247 municipalities).

|  | Linear | Logarithmic | Exponential | Potential |
| --- | --- | --- | --- | --- |
| 247 municipalities | 0.044 | 0.119 | 0.003 | 0.046 |

Table 7. R2 value upon correlating MRW and average travel speed between villages and districts with their municipal (247 municipalities)

|  | Linear | Logarithmic | Exponential | Potential |
| --- | --- | --- | --- | --- |
| 247 municipalities | 0.006 | 0.006 | 0.0074 | 0.0038 |

To continue the analysis of correlations, we proceeded to only work with travel conditions between municipalities and the capital of their respective department. The following regressions were carried out: linear, exponential, potential, and logarithmic between the MRW of each municipality and the average travel time to the capital city (recalling that this variable consists of times georeferenced by Google Maps on a workday and in peak hours). R2 for the department of Quindio varied between 0.40 and 0.97, for Casanare between 0.21 and 0.70, Cundinamarca between 0.17 and 0.33, Meta between 0.026 and 0.29, and for Santander between 0.0004 and 0.025 (Figure 1).

Figure 1. R2 of regressions between MRW and average travel time between various municipalities and the capital city of their department.

Given that average speed in tertiary roadways is usually very similar throughout the entire national territory, when changing the independent variable for travel distance to the capital, R2 values resulted similarly to those analyzed with average times: for Quindio between 0.39 and 0.95, Casanare between 0.16 and 0.70, Cundinamarca between 0.19 and 0.37, Meta between 0.008 and 0.25 and Santander between 0.0002 and 0.017. With these correlations, both with time and distance between municipalities and the capital city of their department, a relationship is discarded between a better economic position of municipalities on a departmental level given that only 2 departments (of 5 analyzed) presented values greater or equal to 0.70 (Figure 1). Therefore, there is no significant relationship between the MRW of municipalities and their respective roadway travel condition connecting to their capital city.

3.2. Analysis according to departmental GDP and roadway conditions

The previous section reports the results of correlating the GDP of 5 departments (of which information was obtained in-field in regards to times, distances, and speeds) with the average conditions of kilometer distance and time between municipalities and their respective villages and districts. The results only validated a correlation for 2 departments. Because of this, it was decided to correlate roadway network variables for 27 of the 32 departments in Colombia, to establish if the departmental GDP is correlated to the total kilometer distance of paved roadways (in the following conditions: Very Good, Good, Regular, Poor and Very Poor) or non-paved roads. The information regarding GDP was obtained from DANE (2020) [28] and information regarding kilometer distance was obtained from INVIAS (2020) [32].

Using departmental GDP 2020 as the dependent variable and departmental paved kilometer distance roadway length in its 5 condition levels as an independent variable (from Very Good to Very Poor), and after conducting the following regressions: exponential, potential, linear and logarithmic, R2 values resulted between 0.005 and 0.06 for the condition Very Good, between 0.05 and 0.18 for Good, between 0.05 and 0.28 for Regular, between 0.05 and 0.22 for Poor, and finally, between 0.008 and 0.11 for Very Poor. Posteriorly, correlation results were studied, however, with the non-paved roadway network as an independent variable. Once again, the following regressions were carried out: exponential, potential, linear, and logarithmic. R2 values resulted between 0.005 and 0.009 for the condition Very Good, between 0.0005 and 0.015 for Good, between 0.00001 and 0.01 for Regular, between 0.00009 and 0.018 for Bad, and finally, between 0.00001 and 0.04 for Very Poor. In both cases, paved and non-paved roadways, R2 values do not reach close to a minimum acceptable value of 0.7 to validate a correlation. Figure 2 shows that R2 values for non-paved roadways are strongly inferior to those of paved roadways, in other words, the relationship is somewhat greater between departmental economy and a paved roadway in comparison to a non-paved roadway. In summary, there is no significant relationship between the global GDP of each department and roadway network length, both in paved and non-paved roadways.

Now, analyzing the previous correlations with R (Pearson correlation coefficient) the highest value obtained was 0.50 (including positive and negative) for the global GDP correlated with the extension of the Paved – Regular roads (Table 8). In summary, there is no significant relationship between the global GDP of each department and roadway network length, both in paved and non-paved roadways. The validated coefficients sign is positive, for both R and R2.

Figure 2. The R2 value for regressions between departmental GDP and roadway network length according to condition:

Regarding coefficient signs in regressions for multiple variables, as for the paved roadway network, these resulted positive, as for the non-paved roadway network, these resulted negative. The above also discards an appropriate statistical behavior, given that length in poor conditions (Poor and Very Poor) (negative sign). However, all signs for the paved roadway network were positive. As for the non-paved roadway network, the effect is similar. In other words, the greater length of the network in good or regular conditions would be expectable to find a greater GDP, and the opposite would be expected for roadways in poor condition; however, the sign in all coefficients resulted being negative. With the above, it is possible to conclude that there is a greater relationship between GDP and a greater length of paved roadway than the one occurring between GDP and non-paved roadway length.

3.3. Analysis according to rural sector GDP and roadway conditions

Given that there was no correlation between roadway network variables and departmental GDP, correlations were proposed, however, only with the components that have a greater impact upon the rural sector: GDP 2020 for the following sectors: agriculture, livestock, hunting, fishing, silviculture (agribusiness) and mining. By adding these records as a dependent variable, we proceeded to carry out correlations with kilometer distance of paved roadway network in its different conditions (Very Good, Good, Regular, Poor, and Very Poor), and do likewise with the non-paved roadway network. Starting the analysis with the paved roadway network and the sum of agribusiness and mining GDP, regressions were carried out for the following models: exponential, potential, linear, and logarithmic. R2 values resulted between 0.0003 and 0.025 for the condition Very Good, between 0.19 and 0.5 for Good, between 0.25 and 0.38 for Regular, between 0.15 and 0.20 for Poor, and finally, between 0.01 and 0.11 for Very poor. Analysis of the non-paved roadway network presents a greater error than the one for the paved roadway network. The greatest R2 value is 0.14 in the condition Very Good, and a linear trend (Figure 3). In summary, the length of roadway network in departments, both for paved and non-paved roadway networks, independent of its condition or state, does not correlate with economic productivity in the agricultural or the mining sector within the territory, represented in sector GDP.

Figure 3. The R2 value for regressions between departmental GDP for the following sectors: agriculture, livestock, silviculture, hunting, fishing, and mining; and roadway network length according to condition.

Given that the correlation between the agribusiness and mining GDP did not result in being statistically valid, it was decided to analyze only the agribusiness GDP, given that these are the activities with greater participation in the Colombian rural economy. Upon conducting these correlations, the greatest R2 value for the following regressions: linear, exponential, potential, and logarithmic, was 0.44 for the condition Good, and potential regression (Figure 4). For the correlation between agribusiness GDP and non-paved roadway network, the resulting R2 values for the following regressions: linear, exponential, potential, and logarithmic, do not exceed the value of 0.03 (Figure 4).

Figure 4. The R2 value for regressions between departmental GDP and the following sectors, agriculture, livestock, silviculture, hunting and fishing; and roadway network length according to condition.

Posteriorly, we decided to apply the correlation, however now with the average annual agribusiness GDP growth (agriculture, livestock, silviculture, hunting, and fishing) between the years 2005 and 2020. Likewise, there was no statistical validity obtained, given that R2 values resulted between 0.0017 and 0.042 for the condition Very Good, between 0.21 and 0.37 for Good, between 0.25 and 0.40 for Regular, less than 0.056 for Poor, and between 0.083 and 0.13 for Very Poor. For the total paved roadway network as an independent variable, potential regression produced an R2 value of 0.37. This means that once again, there is no significant statistical correlation between the tertiary roadway network length of a department and the increase or decrease of agribusiness GDP within the same territory.

Regarding the non-paved roadway, R2 values with the greatest resulting value were for the condition Very Good (0.17), Good (0.12), Regular (0.12), Poor (0.10), Very Poor (0.033), and for the total non-paved-roadway network (0.12). Therefore, neither agribusiness nor agribusiness and mining GDP value, nor the annual rates for both variables, display a trend that statistically validly correlates with departmental roadway network length, according to its state or condition.

Continuing the analysis, the R highest value obtained (including positive and negative) was 0.55 for the agribusiness GDP correlated with the extension of the Paved – Regular roads (Table 8). In summary, reviewing R and R2 values, there is no significant relationship between the economic rural sectors GDP of each department and roadway network length, both in paved and non-paved roadways. The validated coefficients sign is positive, for both R and R2.

Table 8. R-value upon correlating global and sectors GDP and length of tertiary road network in different conditions.

|  | Global GDP | Agrobusiness GDP | Mining GDP | Agrobusiness plus Mining GDP |
| --- | --- | --- | --- | --- |
| Paved - Very good | 0,26 | 0,22 | -0,10 | 0,07 |
| Paved - Good | 0,29 | 0,45 | 0,25 | 0,44 |
| Paved - Regular | 0,50 | 0,55 | 0,26 | 0,50 |
| Paved - Poor | 0,47 | 0,46 | 0,21 | 0,42 |
| Paved Very poor | 0,09 | 0,13 | 0,04 | 0,10 |
| Non-paved - Very good | -0,07 | 0,05 | 0,55 | 0,38 |
| Non-paved - Good | -0,12 | -0,03 | 0,26 | 0,15 |
| Non-paved - Regular | -0,10 | 0,03 | 0,16 | 0,12 |
| Non-paved - Poor | -0,10 | 0,02 | 0,09 | 0,07 |
| Non-paved - Very poor | -0,05 | 0,05 | -0,13 | -0,05 |

3.4. Analysis according to departmental GDP and roadway network classification.

The previous sections analyzed roadway conditions (paved or non-paved in its 5 levels: Very Good, Good, Regular, Poor, and Very Poor). The study continues with the classification of the roadway network (primary, secondary, and tertiary). Among the first inquiries, we wanted to know if the roadway network length of the tertiary roadway network in each of the departments presents any correlation with the territory's territorial extension. Upon carrying out the correlation, with the information provided by the Ministry of Transport (2019), the R2 value resulted in 0.032, which discards a trend in tertiary roadway network density occupying the territory. In other words, the greater the territorial extension, there should be greater the roadway network length.

Posteriorly, we proceeded to correlate variables under the premise that said that the greater length of a tertiary roadway network of a department, said the department would a better economic performance (analyzing GDP). After conducting the following regressions: linear, exponential, potential, and logarithmic, between departmental GDP and tertiary roadway network length in each department the obtained R2 value was (0.28) for linear, (0.09) for exponential, (0.38) for logarithmic (0.09), and (0.40) for potential (Figure 5). A similar analysis was carried out, however, using primary roadway network length, obtaining an R2 value of (0.38) for linear (0.50) for exponential, (0.24) for logarithmic, and (0.53) for potential. The greatest value (0.53) resulted in being less favorable, but close to 0.56 resulting from the analysis between Latin American countries (Urazan et al. 2017) (Figure 5). Given the above, it is deduced that roadway network length in departments does not adequately correlate with the general GDP in each territory.

Figure 5. The R2 value for regressions between departmental GDP, and departmental GDP for the following sectors: agriculture, livestock, silviculture, hunting and fishing, and for Agriculture, livestock, silviculture, hunting and fishing plus mining (dependent variables); with primary and tertiary roadway network length (independent variables)

The GDP of the departmental agribusiness sector was correlated with the tertiary network, with the following results for R2 values: (0.47) for exponential regression, (0.55) for linear, (0.18) for logarithmic, and (0.69) for potential (Figure 5). Upon adding mining GDP to that of agribusiness GDP R2 values for resulted regressions resulted as follows: (0.34) for exponential regression, (0.28) for linear, (0.12) for logarithmic, and (0.67) for potential (Figure 5). R2 values regarding the agribusiness sector (0.69) resulted in a greater value than when including the mining sector (0.67), and very close to 0.7, the reason for which the resulting regression was analyzed (Figure 6) (Ec. 1). Differently to what was concluded in the previous paragraph, the GDP of agribusiness sector does present an acceptable correlation (R2 value close to 0.7) with tertiary roadway network length in each department. Additionally, the standard error results in a value of 0.09 and a p-value of 1.41E-05, confirming that there is a statistically significant correlation.

Figure 6. Potential regression between agribusiness GDP and tertiary roadway network length, according to the department.

The resulting equation (Ec. 1) reiterates the exponent's positive sign, given that with the greater tertiary roadway network length, there is a trend of a greater GDP in the agribusiness sector. Only 2 departments of the 27 analyzed (7.5%) present a graphical position that can be considered as relatively distant from the trend line: Santander (7600, 6985) and Valle (4375, 7142); and are the 2 departments with the greatest GDP in the agribusiness sector.

Ec. 1	 y=52.32*x0.4448

Where y= departmental GDP in the agribusiness sector (thousands of millions COP) and x= departmental tertiary roadway network (km).

Similarly, primary roadway network length was correlated with the agribusiness sector GDP, obtaining R2 values of (0.56) for exponential, (0.57) for linear, (0.41) for logarithmic, and (0.70) for potential (Figure 5). Upon adding mining GDP to agribusiness GDP, R2 values resulted in (0.54) for exponential regression, (0.51) for linear, (0.37) for logarithmic, and (0.73) for potential (Figure 5). Values of the 2 potential regressions resulted in greater or equal to 0.7, providing statistical validity to correlations. Given the above, we proceeded to analyze the results of regressions (Figures 7 and 8; Ecs. 2 and 3), obtaining a good R2 between the primary roadway network of each department and the agribusiness sector GDP in that same territory. Additionally, the standard error is 0.94 (higher than with a tertiary road network) and the p-value is 6.5E-06, which confirms a valid statistical correlation.

Figure 7. Potential regression between agribusiness sector GDP and primary roadway network length according to the department.

The equation resulting from Figure 7 (Ec. 2) reiterates the exponent's positive sign, given that the greater primary roadway network length, there is a trend of obtaining a greater GDP in the agribusiness sector. In this case, it is possible to consider that four departments (greatest agribusiness sector GDP) out of the 27 analyzed (15%) are relatively distant from the trend line: Antioquia (1539, 9948), Cundinamarca (1103, 9350), Santander (966, 6985) and Valle (862, 7142).

Ec. 2	 y=0.2346*x1.4115

Where y= Departmental agribusiness sector GDP (thousands of millions COP) and x= departmental primary roadway network (km).

From the regression that includes mining sector GDP with primary roadway network, Figure 8 and la Ec. 3 are obtained. In this case, Ec. 3 also reiterates the exponent's positive sign, given that the greater primary roadway network length, there is a trend of obtaining a greater GDP in the agribusiness + mining sector. In this case, it is possible to consider that three departments out of the 27 analyzed (11%) are relatively distant from the trend line: Antioquia (1539, 14380), Meta (1019, 17502), and Cauca (1429, 2273). The greatest R2 value (0.73) results from correlating departmental agribusiness sector + mining sector GDP with primary roadway network, which means that primary roadway network length contributes more to mining, agribusiness, livestock, hunting, silviculture, fishing, and mining sectors in different territories or departments within the country in comparison to tertiary roadway network length, despite that the latter allows access to rural zones. In the case of the primary roadway network, the analysis results in a standard error of 1.64 (higher than with only agribusiness GDP) and a p-value of 4.08E-05.

Ec. 3	 y=0.2233*x1.482

Where y= PIB Departmental agribusiness sector GDP + mining sector GDP (thousands of millions COP) and x= departmental primary roadway network (km).

Figure 8. Potential regression between agribusiness plus mining sector GDP and primary roadway network, according to the department.

Expanding the analysis, the statistically significant R values (including positive and negative) are 0.76 for Primary road network correlated to agribusiness sector GDP, 0.74 for Tertiary road network correlated to agribusiness sector GDP, and 0.71 for Primary road network correlated to agribusiness plus mining sector GDP (Table 9). These 3 cases coincide with those that were validated using the R2 values and maintain the correlations with a positive sign.

Table 9. R-value upon correlating global and sector GDP and length of the primary and tertiary road network by the department.

|  | Global GDP | Agrobusiness GDP | Mining GDP | Agrobusiness plus Mining GDP |
| --- | --- | --- | --- | --- |
| Primary Road Network | 0,62 | 0,76 | 0,38 | 0,71 |
| Tertiary Road Network | 0,53 | 0,74 | 0,12 | 0,53 |

At the end of the analysis, the extension of the tertiary road network was correlated with the GDP of the 2 economic sectors that have the greatest participation: commerce (17%) and the manufacturing industry (13%). The agricultural added to the mining sector contribute 11% (annual average from 2005 to 2020) [33].

In a similar manner to the previous analysis, tertiary roadway network length was correlated with the commerce sector GDP, obtaining R2 values of (0.18) for exponential, (0.23) for linear, (0.34) for logarithmic, and (0.22) for potential (Figure 9). The best result (linear case) corresponds to Ec. 4. Upon adding manufacturing industrial GDP to commerce GDP, R2 values resulted in (0.20) for exponential regression, (0.26) for linear, (0.38) for logarithmic, and (0.27) for potential (Figure 10). In this case, the best result (logarithmic case) corresponds to Ec. 5. Values resulted being lower to 0.7, which discards a trend to correlations. Therefore, it is concluded that the development of the tertiary road network in Colombia is related to rural activities and not to the overall regional o departmental economy. However, these two equations are not relevant because R2 is statistically inappropriate, much lower than 0,7.

Figure 9. Logarithmic regression between commerce sector GDP and tertiary roadway network, according to the department.

Ec. 4	 y=2315,2lnx-13269

Where y= PIB Departmental commerce sector GDP (thousands of millions COP) and x= departmental tertiary roadway network (km).

Figure 10. Logarithmic regression between commerce plus manufacturing industry sector GDP and tertiary roadway network, according to the department.

Ec. 5	 y=1997,9lnx-11511

Where y= PIB Departmental commerce plus manufacturing sectors GDP (thousands of millions COP) and x= departmental tertiary roadway network (km).

4. Discussion

There are no frequent studies that analyze a correlation between the quantity and condition of roadway networks of a municipality or department with variables that measure the economic level within a territory. Aiming at this point, this paper contributes to the state-of-the-art, by attempting to correlate travel conditions (time, average speed) and roadway length, with economic development indicators such as departmental Gross Domestic Product and Municipal Relative Weight for municipalities.

In the case of Colombia, and probably in other developing countries, the tertiary road network is the largest in the entire national territory. Between 2005 and 2020, it has grown from 62% to 69% of the total road network and its length has gone from 75.000 to 142.000 km. Additionally, it must be highlighted that 71% of the tertiary road network is administrated with municipal resources [34].

The growing proportion of the tertiary road network is related to a greater contribution to the economy of rural areas because they are the regions to which it provides accessibility. By the very definition, rural areas are tied to agricultural, livestock, and mining economic activities (mainly) and need a connection to roads for their development. The participation of agriculture and mining in the national annual GDP has averaged 11% since 2005 [33], but in rural areas, they are the most important economic sectors. The commerce, education, health, and services sectors (with greater participation in the national GDP) serve the majority of the population in main urban areas (77% in the Colombian case) [35]. Even, it must be said that the manufacturing industry and commerce sectors (13% and 17% of the Colombian GDP) do not represent a valid correlation with the tertiary road network. The highest R2 value is 0.38.

The results obtained in this paper have been developed from Pearson's and Spearman's correlation analysis (R2 and p-value). These methodologies are commonly used to validate or not the hypothetically proposed correlations. The data shows that the management of the tertiary road network depends mainly on rural economic activities and not global economic performance. This conclusion allows future research to focus on the improvements that must be made so that the economic sectors in the country help improve accessibility and quality of life to the inhabitants of the most distant regions through the tertiary road network.

It´s important to mention that if there is a correlation between 2 variables, it doesn´t imply that the independent variable explains the behavior of the dependent variables; that is, there is not necessarily a causal relationship between the 2 variables. Therefore, the explanations of the relationships between variables, exposed in this discussion, stem from social and economic principles and the state of the art and don’t pretend to be supported only by validated statistical correlations.

The study intends that the relationships be statistically validated thoroughly to identify the indicators that require more detailed subsequent studies to establish investment policies in highways that have an economic impact in rural areas.

As previously mentioned, this paper aims to contribute significantly to the state of the art, especially to the infrastructure scenario in Latin America, and ratifies that the development of rural economy requires the adequate extension of the tertiary road network in regions; but the global economies not so much because the highest population density is concentrated in urban areas.

Another aspect to highlight is that the spatial density of the road network is not similar in all regions, therefore the territorial extension does not correlate with rural economic development as the extension of the tertiary road network does.

Linked to the above, nor were statistically adequate the values of R2 resulting in the correlation between the travel condition (average speed and distance) from each municipality to the department capital city, with the economic level (GDP) of those same territorial zones.

Finally, the present study clarifies several beliefs or hypotheses frequently used about the economic and development variables that have a high correlation with the condition of the network of tertiary or rural roads in a territory, at least in Colombia case, and probably in other Latin American countries.

5. Conclusions

This research contributes to the state of the knowledge about the relationship between investment in the road network and economic growth in Latin American countries, specifically in the tertiary or rural road network; a subject of great interest again because agricultural development became essential for the region´s economy.

Based on the results obtained in this study, the conclusions are the following::

- None of the following premises had statistical validity upon being applied to Colombian municipalities: i) having a greater roadway network length in rural areas or better travel conditions measured in average speed, correlates with a better condition in economic sectors; ii) fewer distances and greater average travel speeds between a department's municipalities and its capital city, is correlated with a better economic condition for the municipalities within that department; iii) departments with greater roadway network length (paved or un-paved), and especially, in good conditions, are correlated with better economic conditions.

- Regressions between departmental GDP (dependent variable) and roadway network length, both primary and tertiary (independent variable), do not comply with a statistically valid R2. However, regressions that use agribusiness sector GDP (agriculture, livestock, hunting, fishing, and silviculture) with tertiary roadway network length as an independent variable, presented an R2 of 0.69, a number which validates the relationship between the variables. A similar situation was recorded with the primary roadway network in each department as the independent variable (R2 = 0.70). This is interpreted as an important relationship between a department's economic development (global GDP) and production in the agribusiness sector.

- The mining sector GDP, which makes up part of the development in rural zones, presented a better correlation to the primary roadway network (R2 = 0.73). This is the greatest correlation in the entire study. This allows us to infer that at a departmental level, there is a relationship between the length of a primary roadway network and mining sector production in the respective region. This relationship is more statistically important compared to the relationship in the agribusiness sector.

- The level of development in a municipality, compared to other municipalities in its department, does not correlate with the distance and time from its villages and districts on the roadway.

- If the relationship between tertiary rural network and economic development in the country's departments (measuring the latter in general GDP) likewise, there is no valid correlation.

- However, if the analysis is carried out concerning the GDP of the agribusiness and mining sector, there is a statistically valid correlation obtained (R2 equal to or greater than 0.7). Also, upon correlating sector GDP with the primary roadway network, the resulting R2 value is 0.7 for the primary network and drops to 0.67 for the tertiary roadway network.

- Despite what was expressed in the paragraph above, if the mining sector is eliminated from the analysis and only the agribusiness component is left, the resulting R2 value is 0.7 both for primary and tertiary roadway networks, concluding that activities in rural areas are those which most impact investment in roadway network for a region, in the case of Colombian departments. This explains that the greater economic productivity of departments is in their respective capital city, and the need of extending the roadway network (especially tertiary or rural) is to connect to other municipalities whose productive trend is agribusiness/ mining type. In other words, if there were no municipalities with agribusiness productivity in departments, it would not be importantly necessary to extend their roadway network, the primary network would predominate among department capitals and no tertiary roadway network would be extended with importance.

- Additionally, obtained results establish for the case of Colombia; that there is a valid correlation between the length of a primary and tertiary roadway network with the GDP in the country's departments; ratifying the published study's main conclusion, according to which the GDP of main countries in Latin America and the Caribbean is correlated to the length of their corresponding primary roadway network.

References

- Urazan, C.F., Escobar D.A. y Moncada C.A. (2017). Relación entre la red nacional de carreteras y el desarrollo económico nacional. Caso América Latina y el Caribe. Revista Espacios, vol. 38, No. 61, p.p. 09-22. Obtained from: (http://es.revistaespacios.com/a17v38n61/a17v38n61p09.pdf). DOI: 10.48082/espacios-a17v38n61

- Jacoby, H. G. (2000). Access to markets and the benefits of rural roads. The economic journal, 110(465), 713-737. Obtained from: https://academic.oup.com/ej/article-abstract/110/465/713/5140059.

- Escobal, J., & Ponce, C. (2002). The benefits of rural roads: enhancing income opportunities for the rural poor. Obtained from: http://repositorio.grade.org.pe/bitstream/handle/20.500.12820/233/ddt40EN.pdf?sequence=1&isAllowed=y

- Jalan, J., & Ravallion, M. (2002). Geographic poverty traps? A micro econometric model of consumption growth in rural China. Journal of Applied Econometrics, Forthcoming. Obtained from: https://onlinelibrary.wiley.com/doi/full/10.1002/jae.645.

- Meeserman, H. & Nazemzadeh, M. (2017). The contribution of transport infrastructure to economic activity: the case of Belgium. Case studies on transport policy. http://dx.doi.org/10.1016/j.cstp.2017.03.009

- Howe, J., Richards, P., & Howe, J. D. G. F. (2019). Rural roads and poverty alleviation. Routledge. Obtained from: https://books.google.es/books?hl=es&lr=&id=nK2bDwAAQBAJ&oi=fnd&pg=PT8&dq=rural+roads&ots=ZTWePWXFyI&sig=hq5APxr2R5--gYtkBrBCHZXojdo

- Asher, S., & Novosad, P. (2020). Rural roads and local economic development. American economic review, 110(3), 797-823. Obtained from: https://www.aeaweb.org/articles?id=10.1257/aer.20180268

- Nautiyal, A., & Sharma, S. (2021). Condition Based Maintenance Planning of low volume rural roads using GIS, Journal of Cleaner Production, Volume 312, obtained from: https://www.sciencedirect.com/science/article/pii/S0959652621018679.

- Cortés-Villafradez., R. A. & De la Peña-Cárdenas. (2019). Analysis of transport infrastructure development and competitiveness in the member countries of the Pacific Alliance (2007-2016). Revista Finanzas y Política Económica, 11(2), 277-297. doi: http:// dx.doi.org/10.14718/revfinanzpolitecon.2019.11.2.4

- Palei, T. (2015). Assessing The Impact of Infrastructure on Economic Growth and Global Competitiveness, Procedia, Economics, and Finance. 2nd Global Conference of business, economics, management and tourism, 2014, Prague.DOI: 10.1016/S2212-5671(15)00322-6.

- Bottasso, A.; Conti, M.; Costacurta, P.; Ferrari, P.; Tei, A. (2021). Roads to growth: the Brazilian way. Research in Transportation Economics, https://doi.org/10.1016/j.retrec.2021.101086

- Idei, R., & Kato, H. (2018). Changes in individual economic activities and regional market structures caused by rural road improvement in Cambodia. Transportation research record. Transportation research board, vol. 2672, issue 3. Obtained from: https://journals-sagepub-com.hemeroteca.lasalle.edu.co/doi/full/10.1177/0361198118783863.

- Ghosh, P. & Dinda, S. (2021). Revisited the relationship between economic growth and transport infrastructure in India: an empirical study. The Indian economic journal. Obtained from: https://journals-sagepub-com.hemeroteca.lasalle.edu.co/doi/full/10.1177/00194662211063535

- Magazzino, C. & Mele, M. (2020). On the relationship between transportation infrastructure and economic development in China. Research in transportation economics. DOI: https://doi.org/10.1016/j.retrec.2020.100947

- Batool, I. and Goldmann, K. (2020). The role of public and private transport infrastructure capital in economic growth. Evidence from Pakistan. Research in transportation economics. DOI: https://doi.org/10.1016/j.retrec.2020.100886.

- Rokicki, B. & Stepniak, M. (2018). Major transport infrastructure investment and regional economic development – An accessibility-based approach. Journal of transport geography. DOI: https://doi.org/10.1016/j.jtrangeo.2018.08.010.

- Rothengatter, W. (2017). Wider economic impacts of transport infrastructure investments: Relevant or negligible? Transport policy. DOI: https://doi.org/10.1016/j.jtrangeo.2018.08.010.

- Hansen, W. & Johansen, B.G. (2017). Regional repercussions of new transport infrastructure investments: A SCGE model analysis of wider economic impacts. Research in transportation economics. DOI: https://doi.org/10.1016/j.retrec.2017.07.004

- Consejo Nacional de Política Económica y Social CONPES 3857 (2016). Departamento Nacional de Planeación, República de Colombia.

- Escobar, D.A., Holguín, J.F. y Kaffure, C. (2016). Análisis de la ubicación geoespacial de las instituciones educativas del departamento de Caldas (Colombia) y su relación con las características operativas de la red vial. Revista Espacios, vol. 37, No. 22. Obtained from: https://revistaespacios.com/a16v37n22/16372208.html

- Dagnino, J. (2014). Coeficiente de correlación lineal de Pearson. Chil Anest, 43, 150-153.

- Pando, V. y San Martín R. (2004). Regresión logística multinomial. Cuadernos de la sociedad española de ciencias forestales, No. 18, p.p. 323-327.

- Roy, I., Rivas, R., Pérez, M., y Palacios, L. (2020). Correlación: no toda correlación implica causalidad. Revista Alergia México, vol. 66, No. 3. Obtened from: https://doi.org/10.29262/ram.v66i3.651

- García, M.A. y Silva D.R. (2017). Análisis de la condición de infraestructura de vías terciarias de los departamentos de Casanare y Cundinamarca en Colombia. Trabajo de grado, Universidad de La Salle, Bogotá. 680 p. Obtened from: https://ciencia.lasalle.edu.co/cgi/viewcontent.cgi?article=1124&context=ing_civil

- Carrascal, L. y Cuervo C. (2019). Diagnóstico de la condición de movilidad en vías de conexión veredal en el departamento del Meta. Trabajo de grado, Universidad de La Salle, Bogotá. 218 p. Obtened from: https://ciencia.lasalle.edu.co/cgi/viewcontent.cgi?article=1519&context=ing_civil

- Marín, A. y Fonseca, H. (2021). Análisis de movilidad y caracterización de las vías de conexión entre municipio y vereda del departamento de Quindío, Colombia. Trabajo de grado en desarrollo, Universidad de La Salle, Bogotá.

- Insausti, D. y Pérez, E. (2021). Evaluación del estado de las vías terciarias veredales a partir de la auscultación visual sobre Google Street-view en los departamentos de Santander y Norte de Santander. Trabajo de grado, Universidad de La Salle, Bogotá. 277 p. Obtened from: https://ciencia.lasalle.edu.co/cgi/viewcontent.cgi?article=1942&context=ing_civil.

- Departamento Administrativo Nacional de Estadística DANE (2020). PIB por department. Obtained from: https://www.dane.gov.co/index.php/estadisticas-por-tema/cuentas-nacionales/cuentas-nacionales-departamentales.

- Baker, L. (2018). Beginner´s guide to correlation analysis. 2nd edition. CSI publishing.

- Baker, L. (2018). Correlation is not causation. 2nd edition. CSI publishing.

- Imbens, G. (2021). Statistical significance, p-values, and the reporting of uncertainty. Journal of economic perspectives. Vol. 35, No.  3, p-p.157 – 174. DOI: https://doi.org/10.1257/jep.35.3.157.

- Instituto Nacional de Vías INVIAS (2020). Estado de la red vial. Obtained from: https://www.invias.gov.co/index.php/informacion-institucional/2-principal/57-estado-de-la-red-vial.

- Departamento Administrativo Nacional de Estadística DANE (2020). PIB resultados por actividad económica. Obtened from: https://www.dane.gov.co/index.php/estadisticas-por-tema/cuentas-nacionales/cuentas-nacionales-departamentales#:~:text=Informaci%C3%B3n%202021%20preliminar,de%20millones%20de%20pesos%2C%20respectivamente.

- Ministerio de Transporte. República de Colombia (2021). Transporte en cifras 2020. Obtened from: https://www.mintransporte.gov.co/publicaciones/9443/transporte-en-cifras/

- Departamento Administrativo Nacional de Estadística DANE (2020). Censo nacional de población y vivienda 2018. ¿Dónde estamos? Obtened from: https://www.dane.gov.co/index.php/estadisticas-por-tema/demografia-y-poblacion/censo-nacional-de-poblacion-y-vivenda-2018/donde-estamos.

Appendix A.

- Table A1. Municipalities studied information: MRW, tertiary road network length, and average travel speed.

| Casanare Department Municipalities studied | Municipal Relative Weight (MRW)(%) 2019 | Tertiary road network analyzed (km) | Tertiary road network Average travel speed (km/h) | Average travel speed (km/h) to department capital city |
| --- | --- | --- | --- | --- |
| Aguazul | 11,7 | 214,7 | 22 | 41,54 |
| Chámeza | 0,3 | 48,6 | 20 | 38,40 |
| Hato Corozal | 1,3 | 411,5 | 21 | 61,86 |
| La Salina | 0,1 | 34,1 | 22 | 45,41 |
| Maní | 3,8 | 409,8 | 15 | 52,26 |
| Monterrey | 1,5 | 165,7 | 22 | 44,64 |
| Nunchía | 1,5 | 183,8 | 23 | 46,96 |
| Oracle | 5,5 | 454,7 | 15 | 46,61 |
| Paz de Ariporo | 5,3 | 566,9 | 24 | 56,91 |
| Póre | 1,4 | 161,3 | 17 | 54,68 |
| Receptor | 0,1 | 67,6 | 16 | 39,79 |
| Sabana Larga | 0,4 | 90,5 | 18 | 46,74 |
| San Luis de Palenque | 2,9 | 405,3 | 19 | 60,00 |
| Támara | 0,6 | 120,1 | 11 | 51,11 |
| Tauramena | 16,2 | 265,7 | 20 | 43,33 |
| Trinidad | 2,6 | 315,6 | 16 | 58,43 |
| Villanueva | 11,7 | 190,3 | 25 | 47,79 |
| Yopal | 33 | 412,9 | 20 | 0,00 |
| Cundinamarca Department Municipalities studied | Municipal Relative Weight (MRW)(%) 2019 | Tertiary road network analyzed (km) | Tertiary road network Average travel speed (km/h) | Average travel speed (km/h) to department capital city |
| Agua de Dios | 0,2 | 127 | 16 | 41,33 |
| Alban | 0,2 | 73,4 | 17 | 36,00 |
| Anapoima | 0,9 | 311,2 | 17 | 39,71 |
| Anolaima | 0,3 | 110,5 | 17 | 61,64 |
| Apulo | 0,2 | 92,6 | 18 | 41,22 |
| Arbeláez | 0,4 | 144,5 | 16 | 35,29 |
| Beltrán | 0,1 | 25,1 | 21 | 40,09 |
| Bituimá | 0,1 | 41,1 | 20 | 37,12 |
| Bojacá | 0,3 | 67,1 | 18 | 36,99 |
| Cabrera | 0,3 | 287,5 | 19 | 37,50 |
| Cachipay | 0,4 | 44,4 | 14 | 33,50 |
| Cajicá | 3 | 23,8 | 18 | 41,86 |
| Caparrapi | 0,3 | 373,1 | 22 | 42,00 |
| Cáqueza | 0,7 | 160,7 | 18 | 31,02 |
| Carmen de Carupá | 0,2 | 171 | 19 | 39,45 |
| Chaguaní | 0,1 | 87 | 14 | 34,95 |
| Chía | 5,6 | 38,3 | 18 | 32,09 |
| Chipaque | 0,1 | 171,7 | 20 | 30,33 |
| Choachí | 0,3 | 231,4 | 16 | 36,82 |
| Chocontá | 0,7 | 104,1 | 21 | 59,20 |
| Cogua | 1,1 | 141,6 | 17 | 44,79 |
| Cota | 5,3 | 21,7 | 15 | 25,45 |
| Cucunuba | 0,4 | 105,3 | 19 | 40,91 |
| El Colegio | 0,7 | 87,4 | 15 | 33,00 |
| El Peñón | 0,1 | 18,8 | 17 | 37,37 |
| El Rosal | 0,5 | 53,9 | 21 | 34,91 |
| Facatativá | 5,5 | 132,3 | 16 | 33,85 |
| Fomeque | 0,6 | 428,7 | 15 | 27,19 |
| Fosca | 0,2 | 150,6 | 17 | 30,39 |
| Funza | 5,2 | 20 | 20 | 26,18 |
| Fúquene | 0,2 | 58,1 | 21 | 43,48 |
| Fusagasugá | 3,4 | 334,7 | 16 | 36,48 |
| Gachalá | 0,1 | 75 | 25 | 36,61 |
| Gachancipá | 0,4 | 39,9 | 19 | 54,89 |
| Gacheta | 0,4 | 35,9 | 17 | 43,97 |
| Girardot | 2,9 | 26 | 19 | 43,52 |
| Granada | 0,2 | 20,4 | 16 | 33,49 |
| Guachetá | 0,2 | 198,3 | 17 | 38,85 |
| Guaduas | 0,7 | 91,2 | 20 | 40,22 |
| Guasca | 0,3 | 421,6 | 18 | 48,49 |
| Guataquí | 0,1 | 11,1 | 22 | 49,04 |
| Guatavita | 0,1 | 140,1 | 16 | 55,38 |
| Guayabal de Síquima | 0,2 | 32 | 19 | 34,29 |
| Guayabetal | 0,1 | 79 | 16 | 21,73 |
| Gutiérrez | 0,1 | 138,7 | 17 | 26,41 |
| Jerusalén | 0,1 | 39,8 | 20 | 40,94 |
| Junín | 0,2 | 303,9 | 16 | 43,08 |
| La Calera | 0,9 | 171,6 | 16 | 28,80 |
| La Mesa | 2,1 | 170,4 | 15 | 37,76 |
| La Palma | 0,2 | 66,5 | 22 | 35,32 |
| La Pena | 0,1 | 43,7 | 17 | 36,45 |
| La Vega | 0,5 | 127,2 | 19 | 43,33 |
| Lenguazaque | 0,5 | 127,7 | 20 | 49,46 |
| Machetá | 0,1 | 31,7 | 19 | 55,33 |
| Madrid | 3 | 109,1 | 16 | 27,27 |
| Manta | 0,1 | 18,9 | 17 | 48,75 |
| Medina | 0,2 | 130,4 | 16 | 37,05 |
| Mosquera | 4,5 | 61,7 | 14 | 27,27 |
| Nariño | 0,1 | 18,5 | 13 | 47,18 |
| Nemocón | 0,2 | 78 | 20 | 44,44 |
| Nilo | 0,3 | 129,2 | 15 | 43,12 |
| Nocaima | 0,2 | 14,7 | 14 | 43,02 |
| Pacho | 0,6 | 156,6 | 19 | 39,34 |
| Paime | 0,1 | 113,8 | 18 | 33,19 |
| Pandi | 0,2 | 115,5 | 18 | 39,88 |
| Paratebueno | 0,5 | 187,3 | 20 | 36,50 |
| Pasca | 0,3 | 100,1 | 18 | 34,86 |
| Puerto Salgar | 0,5 | 118,1 | 24 | 47,34 |
| Pulí | 0,1 | 90,8 | 20 | 35,15 |
| Quebradanegra | 0,1 | 48,8 | 18 | 41,48 |
| Quetame | 0,2 | 80,6 | 18 | 35,73 |
| Quipile | 0,3 | 37,8 | 25 | 32,83 |
| Ricaurte | 0,5 | 44,5 | 19 | 43,40 |
| San Antonio del Tequendama | 0,3 | 38,9 | 17 | 33,64 |
| San Bernardo | 0,6 | 103,8 | 21 | 32,86 |
| San Cayetano | 0,2 | 60 | 24 | 29,86 |
| San Francisco de | 0,3 | 28,2 | 16 | 38,71 |
| San Juan de Rioseco | 0,1 | 44,7 | 19 | 39,04 |
| Sasaima | 0,4 | 81,8 | 14 | 38,10 |
| Sesquilé | 0,6 | 60,4 | 21 | 57,00 |
| Sibaté | 1,6 | 62,1 | 19 | 29,21 |
| Silvania | 0,6 | 33,8 | 15 | 57,43 |
| Simijaca | 0,3 | 37,7 | 21 | 46,54 |
| Soacha | 10,3 | 52 | 18 | 25,00 |
| Sopó | 1,6 | 52,6 | 20 | 49,33 |
| Subachoque | 0,3 | 77,2 | 20 | 38,10 |
| Suesca | 0,5 | 91 | 17 | 52,39 |
| Susa | 0,1 | 43,2 | 18 | 46,85 |
| Sutatausa | 0,3 | 25,2 | 18 | 45,31 |
| Tabio | 0,4 | 76,1 | 17 | 36,00 |
| Tausa | 1 | 72,6 | 21 | 46,21 |
| Tena | 0,3 | 40 | 16 | 33,82 |
| Tenjo | 2,2 | 69 | 17 | 32,14 |
| Tibacuy | 0,1 | 65 | 17 | 34,93 |
| Tibiritá | 0,1 | 34,7 | 19 | 52,24 |
| Tocaima | 0,4 | 73,3 | 18 | 38,43 |
| Tocancipá | 7,8 | 40,5 | 19 | 50,67 |
| Topaipí | 0,1 | 16,6 | 15 | 34,79 |
| Ubalá | 1,6 | 59,9 | 19 | 40,10 |
| Ubaque | 0,3 | 48,1 | 15 | 26,42 |
| Ubaté | 0,9 | 87,7 | 17 | 44,73 |
| Une | 0,4 | 107,2 | 16 | 30,25 |
| Útica | 0,1 | 67,5 | 20 | 43,02 |
| Venecia | 0,2 | 45,2 | 16 | 38,34 |
| Vergara | 0,2 | 6,6 | 17 | 41,54 |
| Vianí | 0,1 | 25,7 | 17 | 39,71 |
| Villagómez | 0,1 | 9,5 | 10 | 36,43 |
| Villapinzón | 0,8 | 41,8 | 26 | 65,25 |
| Villeta | 0,7 | 56,3 | 16 | 48,21 |
| Viotá | 0,4 | 51,7 | 16 | 33,54 |
| Yacopí | 0,4 | 136,8 | 21 | 35,60 |
| Zipacón | 0,2 | 29,4 | 16 | 35,17 |
| Zipaquirá | 2,8 | 87,9 | 16 | 42,00 |
| Meta Department Municipalities studied | Municipal Relative Weight (MRW)(%) 2019 | Tertiary roadnetwork analized (km) | Tertiary roadnetwork Average travel speed (km/h) | Average travel speed (km/h) to departament capital city |
| Villavicencio | 22,9 | 51,8 | 28 | 0,00 |
| Barranca de Upía | 0,6 | 213,8 | 50 | 43,56 |
| Cabuyaro | 3,2 | 330,9 | 29 | 48,46 |
| Castilla la nueva | 8,7 | 253,5 | 20 | 40,52 |
| San Luis de Cubaral | 0,2 | 131,1 | 23 | 42,68 |
| El calvario | 0,1 | 24,7 | 20 | 29,29 |
| El castillo | 0,3 | 365,7 | 30 | 44,52 |
| El dorado | 0,1 | 58,3 | 27 | 42,60 |
| Fuente de oro | 1,4 | 327,8 | 43 | 46,30 |
| La Macarena | 0,6 | 59,35 | 19 | 46,94 |
| La Uribe | 0,2 | 19 | 21 | 37,40 |
| Lejanías | 0,9 | 125,5 | 30 | 50,07 |
| Mesetas | 0,4 | 167,1 | 29 | 51,48 |
| Puerto Concordia | 0,4 | 15,6 | 38 | 59,54 |
| Puerto Gaitán | 27,8 | 25,5 | 25 | 62,92 |
| Puerto López | 2,8 | 32,6 | 35 | 60,00 |
| Puerto Lleras | 0,7 | 20,6 | 40 | 50,49 |
| Puerto Rico | 0,9 | 75,9 | 46 | 54,82 |
| Restrepo | 0,6 | 28,3 | 34 | 36,92 |
| San Carlos de Guaroa | 1,2 | 55,3 | 34 | 56,81 |
| San Juan de Arama | 0,7 | 28,1 | 35 | 49,93 |
| San Juanito | 0,1 | 9,04 | 19 | 27,19 |
| Vista hermosa | 0,5 | 32,7 | 28 | 53,13 |
| Santander Department Municipalities studied | Municipal Relative Weight (MRW)(%) 2019 | Tertiary roadnetwork analized (km) | Tertiary roadnetwork Average travel speed (km/h) | Average travel speed (km/h) to departament capital city |
| Bucaramanga | 0,1 | 8,9 | 29 | 0,00 |
| Aguada | 0,1 | 1,9 | 19 | 36,35 |
| Albania | 0,1 | 101,6 | 27 | 40,30 |
| Aratoca | 0,1 | 5,2 | 21 | 37,21 |
| Barbosa | 0,1 | 42,21 | 24 | 42,60 |
| Barichara | 0,1 | 2,55 | 23 | 39,03 |
| Barrancabermeja | 0,1 | 20,7 | 22 | 51,49 |
| Bolívar | 0,1 | 93,1 | 31 | 39,52 |
| Cabrera | 0,1 | 44,8 | 30 | 35,24 |
| Capitanejo | 0,1 | 58,6 | 29 | 31,69 |
| Carcasí | 0,1 | 9,9 | 25 | 29,78 |
| Cerrito | 0,1 | 29,8 | 28 | 32,12 |
| Charalá | 0,1 | 4,7 | 28 | 40,50 |
| Charta | 0,1 | 23,4 | 22 | 29,71 |
| Chima | 0,1 | 31,2 | 22 | 35,18 |
| Chipatá | 0,1 | 67,4 | 27 | 38,89 |
| Cimitarra | 0,1 | 8,6 | 22 | 54,11 |
| Concepción | 0,1 | 18,2 | 27 | 31,73 |
| Confines | 0,1 | 10,9 | 19 | 40,00 |
| Contratación | 0,1 | 42,4 | 27 | 31,40 |
| Coromoro | 0,1 | 74,35 | 23 | 38,32 |
| Curití | 0,1 | 17,5 | 22 | 35,09 |
| El Carmen de Chucuri | 0,1 | 135,9 | 32 | 46,63 |
| El Guacamayo | 0,1 | 37,85 | 32 | 30,60 |
| El Peñón | 0,1 | 67,3 | 23 | 42,63 |
| El Playón | 0,1 | 0,9 | 18 | 36,76 |
| Encino | 0,1 | 33,4 | 21 | 37,79 |
| Enciso | 0,1 | 13,6 | 21 | 31,37 |
| Florián | 0,1 | 6 | 27 | 36,59 |
| Floridablanca | 0,1 | 34,75 | 19 | 33,50 |
| Galán | 0,1 | 13,1 | 20 | 28,49 |
| Gambita | 0,1 | 20,95 | 18 | 41,51 |
| Girón | 0,1 | 47,7 | 25 | 40,00 |
| Guaca | 0,1 | 83,8 | 23 | 29,55 |
| Guadalupe | 0,1 | 26,45 | 24 | 39,92 |
| Guapota | 0,1 | 30,3 | 21 | 40,17 |
| Guavatá | 0,1 | 32,8 | 28 | 42,00 |
| Güepsa | 0,1 | 11,3 | 28 | 42,74 |
| Hato | 0,1 | 95,8 | 18 | 63,48 |
| Jesús María | 0,1 | 78,9 | 27 | 80,67 |
| Jordán | 0,1 | 7,3 | 24 | 33,80 |
| La Belleza | 0,1 | 84,1 | 30 | 37,15 |
| La Paz | 0,1 | 6,6 | 27 | 37,95 |
| Landázuri | 0,2 | 73,1 | 26 | 51,33 |
| Lebrija | 0,2 | 107,2 | 32 | 43,24 |
| Los Santos | 0,2 | 15,3 | 34 | 39,75 |
| Macaravita | 0,2 | 33,6 | 30 | 30,49 |
| Málaga | 0,2 | 6,71 | 43 | 31,12 |
| Matanza | 0,2 | 74,8 | 39 | 30,39 |
| Zapatoca | 0,2 | 29,4 | 26 | 34,58 |
| Mogotes | 0,2 | 37,9 | 26 | 38,79 |
| Molagavita | 0,3 | 37 | 22 | 30,11 |
| Ocamonte | 0,3 | 9,2 | 22 | 40,10 |
| Oiba | 0,3 | 161,3 | 21 | 41,58 |
| Onzaga | 0,3 | 53,9 | 27 | 31,65 |
| Palmar | 0,3 | 50,5 | 28 | 37,25 |
| Villanueva | 0,3 | 78,3 | 34 | 39,23 |
| Palmas del Socorro | 0,3 | 30,1 | 45 | 39,50 |
| Páramo | 0,3 | 135,8 | 30 | 39,44 |
| Piedecuesta | 0,3 | 91,4 | 28 | 46,00 |
| Pinchote | 0,3 | 130,1 | 42 | 39,62 |
| Vélez | 0,3 | 115,8 | 30 | 47,70 |
| Tona | 0,4 | 36,3 | 32 | 27,24 |
| Suratá | 0,4 | 33,2 | 30 | 29,67 |
| Sucre | 0,5 | 563,9 | 30 | 38,73 |
| Suaita | 0,5 | 50,8 | 30 | 42,92 |
| Socorro | 0,8 | 37,4 | 28 | 40,68 |
| Puente Nacional | 0,8 | 99,9 | 34 | 43,22 |
| Puerto Parra | 1 | 66,8 | 33 | 53,26 |
| Simacota | 1 | 27,3 | 31 | 39,23 |
| Puerto Wilches | 1,3 | 13,3 | 32 | 50,77 |
| Santa Bárbara | 1,4 | 46 | 41 | 31,91 |
| San Vicente de Chucurí | 1,4 | 107,5 | 36 | 40,27 |
| San Miguel | 1,5 | 130,9 | 30 | 118,29 |
| Rionegro | 1,7 | 111,2 | 38 | 33,29 |
| Sabana de Torres | 4 | 181,2 | 35 | 54,24 |
| San José de Miranda | 5,2 | 228,5 | 29 | 31,28 |
| San Andrés | 7,2 | 47 | 31 | 30,74 |
| San Benito | 8,3 | 60,2 | 28 | 39,52 |
| San Gil | 26,5 | 324,319048 | 23 | 38,67 |
| San Joaquín | 27,2 | 486,2 | 44 | 33,72 |
| Quindio Department Municipalities studied | Municipal Relative Weight (MRW)(%) 2019 | Tertiary roadnetwork analized (km) | Tertiary roadnetwork Average travel speed (km/h) | Average travel speed (km/h) to departament capital city |
| Armenia | 54 | 5,02 | 25 | 0,00 |
| Buenavista | 1 | 4,02 | 20 | 36,88 |
| Calarcá | 9 | 8,36 | 15 | 22,67 |
| Circasia | 4 | 4,39 | 17 | 33,39 |
| Córdoba | 1 | 3,25 | 15 | 35,60 |
| Filandia | 3 | 3,58 | 22 | 40,00 |
| Génova | 1 | 5,06 | 16 | 35,93 |
| La Tebaida | 8 | 5,85 | 28 | 42,00 |
| Montenegro | 9 | 5,96 | 26 | 35,40 |
| Pijao | 2 | 3,42 | 15 | 33,20 |
| Quimbaya | 7 | 5,98 | 26 | 36,00 |
| Salento | 2 | 6,73 | 24 | 38,05 |

- Table A2. Colombia departments were analyzed by adding municipalities’ information. GDP 2020 in thousands of millions COP.

| Department | Global GDP 2020 | Tertiary roadnetwork Average travel speed (km/h) | Tertiary roadnetwork analized (km) | Average travel speed (km/h) to departament capital city |
| --- | --- | --- | --- | --- |
| Casanare | 13.121 | 19 | 4519 | 48,86 |
| Cundinamarca | 61.644 | 18 | 10526 | 38,77 |
| Meta | 31.363 | 31 | 2452 | 46,63 |
| Santander | 62.570 | 28 | 5510 | 39,02 |
| Quindío | 8.303 | 21 | 62 | 35,88 |

- Table A3. Colombia’s department’s road network length by the condition in 2020.

| Department | Paved (Km) |  |  |  |  | No paved (Km) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Very Good | Good | Regular | Poor | Very Poor | Very Good | Good | Regular | Poor | Very Poor |
| Antioquia | 51,37 | 122,24 | 222,38 | 227,11 | 1,00 | 0,10 | 0,25 | 7,73 | 0,10 | 0,10 |
| Atlántico | 18,75 | 4,81 | 2,49 | 0,10 | 0,10 | 0,10 | 3,47 | 8,30 | 28,08 | 2,90 |
| Bolívar | 15,98 | 74,94 | 48,40 | 15,78 | 0,10 | 0,10 | 2,55 | 0,10 | 0,10 | 0,10 |
| Boyacá | 41,44 | 196,93 | 288,15 | 145,44 | 2,00 | 0,10 | 1,21 | 43,64 | 123,85 | 0,09 |
| Caldas | 72,24 | 89,70 | 18,01 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 |
| Caquetá | 138,70 | 90,23 | 83,06 | 86,31 | 0,10 | 0,10 | 29,24 | 15,05 | 11,87 | 0,10 |
| Casanare | 1,93 | 154,76 | 222,38 | 182,77 | 0,10 | 0,17 | 20,12 | 18,80 | 8,60 | 0,10 |
| Cauca | 105,01 | 176,50 | 243,30 | 118,38 | 1,15 | 2,11 | 97,19 | 265,69 | 259,88 | 9,40 |
| Cesar | 88,08 | 154,82 | 98,80 | 103,86 | 15,98 | 0,10 | 0,10 | 1,00 | 23,90 | 3,49 |
| Choco | 35,72 | 80,78 | 50,99 | 3,78 | 0,10 | 0,10 | 0,10 | 52,43 | 34,03 | 20,45 |
| Córdoba | 51,32 | 33,21 | 68,36 | 90,14 | 0,10 | 2,44 | 13,65 | 13,01 | 16,92 | 0,10 |
| Cundinamarca | 6,41 | 69,54 | 69,95 | 64,03 | 0,10 | 0,10 | 0,58 | 17,37 | 11,27 | 0,10 |
| Guajira | 34,44 | 74,64 | 26,75 | 12,91 | 0,10 | 0,10 | 6,71 | 3,55 | 0,10 | 0,10 |
| Huila | 35,01 | 94,39 | 76,42 | 65,15 | 0,97 | 0,10 | 23,52 | 105,36 | 84,43 | 0,10 |
| Magdalena | 39,58 | 75,89 | 20,32 | 8,19 | 14,47 | 0,10 | 0,10 | 18,02 | 34,66 | 36,82 |
| Meta | 46,02 | 173,31 | 120,62 | 11,85 | 0,88 | 4,05 | 45,63 | 103,44 | 79,08 | 0,10 |
| Nariño | 169,25 | 300,27 | 142,33 | 93,00 | 0,38 | 2,25 | 0,10 | 0,10 | 2,36 | 10,35 |
| Norte de Santander | 19,98 | 140,90 | 135,20 | 131,04 | 0,10 | 0,10 | 0,10 | 72,40 | 34,76 | 6,00 |
| Putumayo | 97,74 | 46,31 | 5,75 | 4,85 | 0,10 | 0,10 | 9,99 | 43,26 | 75,79 | 0,10 |
| Quindío | 39,99 | 30,37 | 40,35 | 2,00 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 |
| Risaralda | 13,90 | 95,05 | 76,41 | 25,09 | 0,10 | 0,10 | 0,10 | 17,42 | 16,46 | 2,98 |
| Santander | 120,53 | 369,26 | 273,03 | 95,29 | 15,66 | 0,10 | 9,88 | 45,77 | 78,23 | 27,10 |
| Sucre | 35,00 | 24,54 | 32,94 | 45,26 | 14,95 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 |
| Tolima | 5,85 | 133,06 | 46,56 | 20,54 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 |
| Valle | 241,10 | 205,87 | 252,64 | 70,22 | 6,66 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 |
| San Andrés y Providencia | 12,00 | 10,50 | 15,00 | 7,80 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 | 0,10 |

- Table A4. Colombia departments GDP 2020: Global and some sectors. Values in thousands of millions COP.

| Department | Global | Manufacturing industry sector | Commerce sector | Agrobusiness sector | Mining sector |
| --- | --- | --- | --- | --- | --- |
| Antioquia | 149,666 | 20,586 | 23,856 | 9,948 | 5,032 |
| Atlántico | 44,923 | 6,666 | 8,766 | 491 | 104 |
| Bolívar | 34,501 | 5,298 | 5,045 | 1,869 | 890 |
| Boyacá | 27,214 | 3,039 | 4,416 | 3,528 | 1,599 |
| Caldas | 17,034 | 1,957 | 2,757 | 2,247 | 421 |
| Caquetá | 4,181 | 108 | 687 | 662 | 13 |
| Casanare | 13,121 | 368 | 2,514 | 1,809 | 4,788 |
| Cauca | 18,245 | 2,951 | 2,038 | 2,583 | 190 |
| Cesar | 16,812 | 607 | 2,050 | 1,814 | 4,619 |
| Chocó | 4,526 | 31 | 479 | 883 | 930 |
| Córdoba | 18,167 | 1,940 | 2,336 | 2,172 | 751 |
| Cundinamarca | 61,644 | 12,202 | 9,281 | 9,350 | 411 |
| Guajira | 8,093 | 61 | 1,236 | 527 | 1,484 |
| Huila | 16,810 | 556 | 2,695 | 3,397 | 805 |
| Magdalena | 13,760 | 518 | 2,615 | 2,498 | 33 |
| Meta | 31,363 | 770 | 3,739 | 4,413 | 13,089 |
| Nariño | 15,838 | 358 | 2,952 | 2,749 | 135 |
| Norte de Santander | 15,798 | 804 | 2,741 | 1,811 | 374 |
| Putumayo | 3,331 | 29 | 564 | 247 | 646 |
| Quindío | 8,303 | 360 | 1,643 | 1,603 | 19 |
| Risaralda | 16,605 | 2,030 | 3,283 | 1,335 | 82 |
| Santander | 62,570 | 10,208 | 8,410 | 6,985 | 1,886 |
| Sucre | 8,444 | 537 | 1,329 | 843 | 50 |
| Tolima | 21,621 | 1,896 | 3,459 | 3,905 | 598 |
| Valle | 100,169 | 15,614 | 16,588 | 7,142 | 126 |
| San Andrés y Providencia | 1,312 | 17 | 688 | 25 | 1 |

- Table A5. Colombia department GDP 2020 and primary, and tertiary roadway network length.

| Department | Global GDP Thousands of million COP | Tertiary roadway network (km) | Primary roadway network (km) |
| --- | --- | --- | --- |
| Antioquia | 149,666 | 12500 | 1.539,00 |
| Atlántico | 44,923 | 1250 | 340,00 |
| Bolívar | 34,501 | 6250 | 548,00 |
| Boyacá | 27,214 | 14000 | 1.054,00 |
| Caldas | 17,034 | 2500 | 378,00 |
| Caquetá | 4,181 | 4900 | 440,00 |
| Casanare | 13,121 | 3750 | 768,00 |
| Cauca | 18,245 | 6875 | 1.429,00 |
| Cesar | 16,812 | 1875 | 723,00 |
| Choco | 4,526 | 625 | 270,00 |
| Córdoba | 18,167 | 6875 | 749,00 |
| Cundinamarca | 61,644 | 13125 | 1.103,00 |
| Guajira | 8,093 | 2250 | 357,00 |
| Huila | 16,810 | 8125 | 838,00 |
| Magdalena | 13,760 | 4375 | 549,00 |
| Meta | 31,363 | 5000 | 1.019,00 |
| Nariño | 15,838 | 5000 | 793,00 |
| Norte de Santander | 15,798 | 3700 | 667,00 |
| Putumayo | 3,331 | 1200 | 399,00 |
| Quindío | 8,303 | 1875 | 138,00 |
| Risaralda | 16,605 | 1900 | 300,00 |
| Santander | 62,570 | 7600 | 966,00 |
| Sucre | 8,444 | 3000 | 371,00 |
| Tolima | 21,621 | 9375 | 877,00 |
| Valle | 100,169 | 4375 | 862,00 |
| San Andrés y Providencia | 1,312 | 0,05 | 50,00 |