Environmental Application of Biogenic Magnetite Nanoparticles to Remediate Chromium (III/VI)-Contaminated Water

Abstract

Biogenic minerals’ physicochemical characteristics, such as their high specific surface areas and high reactivities in addition to the presence of a bacterial carrier matrix, make them promising for various applications, for instance, catalysts, adsorbents, oxidants, or reductants. The objective of this study was to examine the efficiency of biogenic magnetite nanoparticles (BMNs) produced by metal-reducing bacteria for removing chromium. Interactions between ionic chromium (Cr III/VI) and BMNs were examined under different solutions pH (pH ranging from 2 to 12) using different BMN doses (0–6 g/L). Chemically synthesized magnetite nanoparticles (CMNs) were used in experiments for comparison to the BMNs. Results showed that BMNs had higher Cr(VI) removal efficiency (100%) than CMNs (82%) after a 2-week reaction time. A lower pH and longer reaction time in the Cr-contaminated solution led to a higher Cr(VI) removal efficiency. The Cr(VI) removal efficiency by BMNs in Cr-contaminated groundwater was about 94% after a reaction time of two weeks. BMNs coated with organic matter were more effective than CMNs in leading to adsorption of Cr(III) with electrostatic interactions (82% versus 13%) and in preventing Fe(II) oxidation within the magnetite structure. These results indicate that BMNs could be used to decontaminate ionic Cr in environmental remediation technologies.